- Cost Estimate

  - Introduction and Assumptions

Detailed cost estimate for Chenani-Sudhmahadev road section has been finalised based on the improvements proposed under Chapter – 8. The detailed estimate is worked out based on the quantities calculated for the items of work to be executed in the project and rates derived after detail analysis and as contained in the government Basic schedule of Rates.

Following assumptions have been made for calculating quantities, rate analysis and cost estimate.

- It is assumed that suitable water would be available for construction purpose within reasonable lead and hence no separate haulage / rate has been considered for this purpose.

- Establishment of good hygienic labour camp is deemed to be included in adopted rates and hence no separate provision has been made.

- Establishment of field laboratory for conducting basic tests on soils, construction material and for quality control is also deemed to be included in adopted rates.

- For road work, bituminous construction, bridge work and CD work, basic lead of 5 km is considered for all completed items and thereafter additional lead component has been considered.

- All sundries, contractor profit, and other overhead charges are deemed to be included in the derived rates. Items required for adhering to safety standards during construction and maintenance phases mentioned in O&M standards are also deemed to be considered.

- Mechanised construction using hot mix batching plant, pavers, concrete batching plant etc has been assumed while working out the rates.

  - Adoption of Unit Rates

The cost estimate of the project road as presented in the Final DPR is based on the final development proposals and priced at latest schedule of rates of Jammu and Kashmir.

The cost estimate has been done with the consideration that the full proposed length of the road will be constructed in one section under one construction package.

For arriving at unit rates at Feasibility stage, it has been assumed that the specifications generally conform to the provisions made in ''Specifications for Road and Bridge Works (IVth Edition)'' of MORT&H.

To develop a thorough understanding of the prevailing construction rates the Consultant have reviewed Basic Schedule of Rates (BSR) published by Public Works Department, Jammu and Kashmir year 2012 including 20% escalation.

    - Based on Rate Analysis

The consultant has adopted the rates from Schedule of rates, 2012 of Road Construction Department, Government of Jammu and Kashmir for major items of works including 20% escalation.

    - Based on Market Rates

The consultant has thoroughly reviewed the market rate and adopted the market for the items of works such as Bitumen, steel, cement etc.

  - Bill of Quantities for Civil Works

The quantities of major items of works have been worked out based on the preliminary highway design, inventory, condition surveys, and other pavement investigations data. The pavement quantities have been worked out based the geometrics and cross sections, pavement design done based on traffic and laboratory investigations.

Site Clearance:

The area considered for Site Clearance is the area within the proposed Right of Way minus the existing carriageway area.

Earth Works:

This item provides for roadway excavation, earthwork in embankment, subgrade and shoulders including disposal of surplus earth and unsuitable material. The earth work quantities like roadway in embankment have been computed based on the data collected during inventory survey. The quantity for cutting in deep section is computed and further classified as cutting in ordinary rock or cutting by open/ controlled blasting in hard rock. The earthwork quantities are based on our site surveys and highway design. Sub-grade having a CBR > 9% will be taken from borrows area.

Sub-base, Base, Surface Courses:

These provide for the items of GSB and WMM for the main carriageway. The quantities for road pavement, base, sub-base etc. for main carriageway have been calculated through applicable cross-sectional template developed in excel software. A provision for cross-fall correction layer has been made for existing carriageway and its quantity has been worked out.

Bituminous Works:

Flexible pavement has been considered for the project road. Bituminous works provide for all items of bituminous courses and surfacing. Quantities for the pavement component are based on the pavement designs proposed in Chapter 8.

Culverts:

The estimation of quantities for culverts was based on site inventory condition survey and study of require hydraulics. The detailed recommendations are given in Chapter 8. The quantities for structures have been calculated based on detailed General Arrangement Drawings (GAD) and other associated drawings using STAAD software and in-house software.

Bridges and structures:

The cost for bridges has been worked out based on the quantities derived from per sqm rate.

Junctions Improvement:

This item includes quantities of kerbs, railings, median etc. The cost for junctions also includes the cost for ‘at grade’ junctions, which need improvement along the highway.

Traffic Signs and Markings:

Proper traffic signs were planned at required locations along the project corridor. It is reviewed considering the traffic and pedestrian safety. The number of traffic signs shall be adequate and modified if required. Centre line and edge markings required from safety point of view were considered in the quantity estimate. RCC Guard posts, double sided metal beam barrier and pedestrian steel guards have been considered at appropriate locations.

Drainage and Protection works:

Provision under this sub-head has been made for surface, subsurface and roadside drains, drainage chutes in cement concrete and stone pitching at outfalls/escapes for drainage. This covers for unlined, open lined and covered drains. The quantities for drainage, protection of embankment & protection against tank bund and river training works are computed based on typical drain drawings and tentative drainage plan.

Miscellaneous Items:

A lump sum amount has been provided for project house, furniture and equipment required for project maintenance, parking, footpath, electrifications, and roadside amenities. In addition to these, traffic control and diversion, bus-stops and cross utility ducts have also been provided.

Utility Shifting

Broad provision is made in the cost estimate for raising and or shifting high-tension lines, electric supply lines, telephone lines and other utilities.

- Table 9.1: Description of Bills for Cost Estimate

| Major Heading | Item of Works |
| --- | --- |
| Site Clearance | - Clearing and Grubbing - Dismantling of existing structures/km stones/ pavement/ road signs - Cutting of Trees and Removal of stumps - Scarifying existing bituminous surface - Dismantling |
| Earth work | - Earthwork in excavation for Ordinary soil / soft rock / hard rock - Embankment construction with material from borrow area - Embankment construction with material from road cutting - Subgrade and Shoulder construction - Turfing |
| Non-Bituminous Courses | - Granular sub-base - Wet mix macadam - Footpath |
| Bituminous Course | - Prime coat - Tack coat - Bituminous Macadam as Profile Corrective Course - Dense Bituminous Macadam - Semi dense bitumen macadam - Bituminous Concrete |
| Cement Concrete Pavement | - Dry Lean Concrete - Pavement Quality Concrete |
| Bridge and Cross Drainage Structures | - Earthwork in excavation for Ordinary soil / soft rock / hard rock - Concrete work in foundation, substructure and superstructure - CR masonry work in foundation and substructure - Slab culvert (widening / new construction / repair / on cross road) - Pipe culvert (widening / new construction / repair / on cross road/ duct for utility crossing) - Major/Minor Bridge (widening / new construction / repair) - RCC bore pile and pile cap - Load test of Pile - Reinforcement in foundation, substructure and superstructure - HT Steel - Steel liner - Bearing - PTFE, Tar paper, elastomeric - Expansion joint - Strip seal, Pre-moulded filler - Asphaltic Wearing coat - Cement paint to exposed concrete - PMC mortar & epoxy bonding coat to concrete - Stone pitching in slope and apron - NP-4 Pipe for culvert |
| Drainage and Protection works | - Unlined drain - Covered lined drain - Chute Drain - Pitching - Reinforced Earth Structure - Stone pitching - Inspection Chamber/ Catch pits - Filter media - Reinforced Earth Structure |
| Road side Furniture | - Km. stone / Boundary stone - Road signs - Pavement markings - Road signage - Crash Barrier - Road stud - Railing - Fencing - Kerb |
| Maintenance | - Diversion - Routine Maintenance |
| Electrical Works | - Streetlight in Urban area - Lighting at toll plaza - Lighting at Truck Lay byes - Lighting at Intersections |
| Miscellaneous Items | - Road side Barriers |
| Way side amenities | - Utility duct - Bus shelter - Tree plantation |
| Toll Plaza | - Toll Booth - Barrier Gate - Canopy - Administrative Building |
| HTMS | - Highway Traffic Management System |

  - Costing for Safety Devices

The safety devices have been proposed based on criteria given in Chapter 9 – Improvement Proposal. Cost for safety devices like crash barrier, road signs and markings, delineators, kerbs, etc. have been derived in Bill of Road Side Furniture.

  - Land Acquisition Cost

Area of land acquisition has been derived based on the actual area calculation from the plan drawing which is difference of proposed ROW and existing ROW. The land acquisition cost has been derived cost estimate summary.

  - Cost of R&R

- A tentative R & R cost (i.e. cost for acquisition of land & structures, resettlement site development, transitional allowance, staff training, and institutional arrangement & strengthening etc,) is expected to be about Rs. 26.15 Crores.

  - Total Cost Estimate

The Abstract and Detailed cost estimate is presented and summarised the chapter.

The summary of cost estimate has been computed and presented in Table 9.2 below.

- Table 9.2: Section wise Cost of Civil Works

| Section | Proposed Length (km) | Base Cost In crore |
| --- | --- | --- |
| Chenani-Sudhmahadev (Existing Ch. 0/000 to 22/713) | 16.990 | 174.34 |

| General Abstract of Cost |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sr No | Detail | Unit | No | Length | Rate | Cost |
| 1 | Road Work |  |  |  |  |  |
|  | Site Clearance |  |  |  |  | 8,791,581 |
| (a) | Typical Cross Section Type- 1 | Km | 1 | 6.902 | 18,053,248 | 124,603,515 |
| (b) | Typical Cross Section Type- 2 | Km | 1 | 0.451 | 17,774,328 | 8,016,222 |
| (c) | Typical Cross Section Type- 3 | Km | 1 | 0.800 | 17,543,793 | 14,035,034 |
| (d) | Typical Cross Section Type- 4 | Km | 1 | 1.000 | 22,766,894 | 22,766,894 |
| (e) | Typical Cross Section Type- 5 | Km | 1 | 1.676 | 17,774,328 | 29,789,774 |
| (f) | Typical Cross Section Type- 6 | Km | 1 | 5.734 | 18,782,826 | 107,700,722 |
| (g) | Extra Widening | Km | 1 | 6.701 |  | 8,353,498 |
| (h) | Cutting & Filling & L&R |  |  |  |  | 236,005,024 |
|  | Total Road Works |  |  |  |  | 560,062,265 |
| 2 | Culvert |  |  |  |  |  |
| (i) | Construction of Culvert |  |  |  |  | 168,223,193 |
|  | Total Culvert Works |  |  |  |  | 168,223,193 |
| 3 | Bridges |  |  |  |  |  |
| (i) | Construction of Major Bridges |  |  |  |  | 455,501,351 |
| (ii) | Construction of Minor Bridges |  |  |  |  | 85,112,278 |
|  | Total Bridge Works |  |  |  |  | 540,613,629 |
| 4 | Other Works |  |  |  |  |  |
| (i) | Drainage & Protection Works |  |  |  |  | 431,036,199 |
| (ii) | Traffic Signs |  |  |  |  | 26,921,860 |
| (iii) | Project Facilities |  |  |  |  |  |
| a) | Bus Bay | No | 4 |  | 649,721.49 | 2,598,886 |
| (iv) | Diversion During Construction |  |  |  |  | 4,561,288 |
| (v) | Junction |  |  |  |  |  |
| a) | Major Junction |  |  |  |  | 1,904,141 |
| b) | Minor Junction |  |  |  |  | 2,535,189 |
| (vi) | Miscellaneous |  |  |  |  | 4,983,267 |
|  | Total Other Works |  |  |  |  | 474,540,829 |
|  | Grand Total |  |  |  |  | 1,743,439,916 |
|  | Cost Per Km |  |  |  |  | 10.26 |