- Financial Analysis

Financial Study is carried out for Chenani-Sudhmahadev road from existing km 0+000 to km. 22+713.

This section is of 16+990 km. proposed length. The commercial viability is assessed for 15 years concession period by making financial analysis (BOT / Annuity analysis) as given below.

Alternative I   - Construction Cost – 174.34 at 15 Years Concession Period

For above alternatives viability has been assessed for all possible modes i.e. BOT & PPP.

  - Approach

The viability of any BOT / Annuity package depends on working cash flows available to service the debt and equity. This working cash flow is basically dependent upon the following:

- Capital Cost

- Traffic Forecast

- User fee Structure

- Operation and Maintenance expenses

- Interest on Debt

- Tax

Infrastructure projects are typically capital intensive and are characterised by long payback periods. To look at such projects on a commercial format, it becomes necessary to adopt measures, which significantly improve the financial viability of the project. Such steps include optimising capital costs and drawing up a user fee structure based on benefit analysis and revenue optimisation principles.

In this ultimate analysis, the extent to which such projects raise non-budgetary resources depend on the ability of the project to service investments at commercial terms. Presently, average returns on equity are structured to range from 12% to 13% per annum on an IRR basis. Likewise, debt instruments placed with financial institutions provide an average return of around 9% to 12% per annum. To raise the resources for such project, it is necessary that the project is expected to give the lucrative returns at healthy debt-equity ratio.

The main objective of Financial Analysis is to examine the viability of implementing the project on a BOT / PPP basis. The analysis attempts to ascertain the extent to which the investment can be recovered through toll revenue and the gap, if any, be funded through Grant / Subsidy. This covers aspects like financing through debt and equity, loan repayment, debt servicing, taxation, depreciation, Annuity etc. The viability of the project is evaluated on the basis of Project FIRR (Financial Internal Rate of Return on total investment). The FIRR is estimated on the basis of cash flow analysis, where both costs and revenue have been indexed to take account of inflation. Financial analysis has been carried out for debt equity ratio of 70:30.

  - Capital Cost

The construction is expected to be executed completely in 24 months; year-wise progress will be 50% in first year, 50% in subsequent year.

    - Base Cost

The civil construction cost for the project road is considered as base cost. The construction is expected to be executed completely in 24 months; year-wise progress will be 50% in first year, 50% in subsequent year. Total Project cost showing these additional provisions are tabulated below:

Table 11.1: Phasing of Cost

| Section | Proposed Length (km) | Base Cost (Rs.) | Phasing of Cost |
| --- | --- | --- | --- |
| I | 16.990 | 174.34 | 50% Cost in 1st year, 50%cost in 2nd year |

    - Cost Escalation

The price escalation provision assumes an increase of 5% per annum on construction cost based on 2012 rates. This is in line with conventional trend of inflation rates.

    - Interest During Construction

A Debt Equity ratio of 70:30, which is commonly used, for infrastructure projects, has been considered for financial analysis of this project. For the loan repayment in Annuity, a repayment schedule of 15 years has been adopted. The interest on long term debt has been taken as 11.75%. The rate for calculation of IDC has also been taken as 11.75%.

    - Landed Project Cost

In base construction cost, provision for physical contingency (2.8% of the base construction cost) has been made to arrive at the Engineering procurement cost (EPC). In addition to the above, a provision of 3% Agency Charges, 0.25% Quality Control Charges, 0.25% Road Safety Charge, 3% Supervision Charges and 5% Escalation charges on total cost have been made to get the project cost.

This financial implication increases the Total Project Cost by approximately 25% of Civil Construction Cost.

| Sr. No. | Item of Works | Cost (cr.) |
| --- | --- | --- |
| A | Civil Works | 174.34 |
|  | Civil Construction Cost (A) |  |
| B | Financial Cost (B) | 56.32 |
| Total Project Cost (A + B) |  | 230.66 |

  - Operation and Maintenance Costs

All Operating and Maintenance costs are to be increased annually by 5%, the same as the present increment rate for WPI. Routine maintenance cost is taken at the present rate of Rs 1.5 lacs per km per annum for 2-lane road. Periodic maintenance cost for 2 lanes has been calculated on the actual rate and quantities,

It is assumed to provide renewal course at every 5th year interval.

For estimating Toll Plaza operation expenses, a team of collection personnel consisting; supervisor, booth manager, service engineer, accountant and security guard has been considered. Apart from their salaries, other overheads for various expenditure viz. communications, electricity, conveyance, furnishings and exigency charges etc. have been considered. The cost towards maintaining regular insurance cover till the end of concession period has also been incorporated. User fee plaza operation cost per plaza for a six lanes booth system is considered 7.5% of expected revenue.

  - Project Revenue

Infrastructure project like Highways are generally having two types of revenue generation.

- Toll Revenue

- Advertisement Revenue

    - Toll Revenue

      - Basic Toll Rates

In assessing the financial viability of a user fee model road project, the willingness of people to pay user fees is a key issue. The standard approach to this issue is to estimate the generalised or perceived cost of trip making. To standardise the toll structure MoRT&H has recommended that basic rate per km for National Highways are adopted for analysis purpose.

Table 11.3: Official Toll Rates April 2007-08

| Vehicle Type | Two lane Carriageway (Base Fee Rate) per km |
| --- | --- |
| Car | 0.39 |
| Mini Bus/LCV | 0.63 |
| Bus/ 2 axle | 1.32 |
| 3 - Axle / MAV | 2.07 |
| Oversize Vehicle | 2.52 |

The base rates given in Table 11.3 shall be increased without compounding by three percent each year and such increased rate shall be deemed to be the base rate for next year. Such applicable base rate shall be revised annually to reflect the increase in wholesale price index; but restricted to forty percent of the increase in wholesale price index.

The toll rates are allowed with 10% inflation at every two years interval and shall be rounded off to nearest 5 rupees. User fees are revised once in every two years.

However, no toll plaza is proposed in the road stretch.

      - Location of Toll Plazas

No toll plaza is proposed for the entire length of the road.

      - Traffic Assumptions and Forecast

Light vehicles especially Two/Three wheelers and slow-moving vehicles are exempted from user fee as per guide lines for toll roads. Tollable traffic has been assessed leaving slow moving and light vehicles except light commercial vehicles, car, mini buses, trucks and Tractor with trailers.

To workout tollable traffic for toll plaza continuous traffic volume count survey has been carried out at 1 location for seven days. To calculate future traffic growth rate of 5% for has been applied to base year tollable traffic.

Projected Traffic figures worked out at toll plaza locations is attached in Annexure 11.1.

      - Concession Period

The guiding principle for determining project specific concession period is the carrying capacity of the respective highway at the end of the proposed concession period. As such, the concession period is proposed based on volume of present and projected traffic. In other ways concession period ends in the year when capacity of respective highway exhaust to cater project traffic volume.

Table provided below shows existing traffic and the year when traffic exceeds the capacity of the highway, calculated for ''Level of Service B (LOS B) and warrants capacity augmentation.

Lane Capacity and Augmentation Required for the Project Corridor

| Homogeneous Sections | LOS B | LOS C |
| --- | --- | --- |
|  | 2-Lane with Paved Shoulder | 2-Lane with Paved Shoulder |
| Chenani-Sudhmahadev (Ch.0+000 to km 22+713) | 2018 | Beyond Analysis Period |

It is revealed from the capacity analysis results; the project road requires 2 lanes with paved shoulder for capacity augmentation and efficient movement of traffic up to project common concession period of 15 years i.e. horizon year 2035.

    - Advertisement Revenue

The advertisement has been not recommended on the project road due to safety concern to the road users. So, revenue generation from the advisement is not taken in to consideration for this project.

  - Financial Model Input and Analysis

    - Grant and Its Treatment

It shall be equal to the sum specified in the bid and as accepted by the authority but in no case greater than the equity and shall be further restricted to a sum not exceeding 40% of TPC i.e. maximum 40% of TPC (Total Project Cost). It shall be due and payable to the Concessionaire as per PPP guidelines. Further it shall be disbursed proportionately along with the loan funds.  For analysis, VGF of 40% have been adopted to ensure minimum 12% IRR.

    - Proposed Sources of Finance

In general, the developer shall crystallize the sources of finance by optimizing his equity returns keeping in view the project cash flows, terms, and conditions of various financing options available. Further the market standing and financial strength of the Developer would largely determine the terms and conditions of finance offered to the Developer by various lending agencies. For the study, following sources of finance have been taken:

- Equity: To be provided by the Developer

- Subsidy / Grant for viability of funding, to be provided by the client.

- Debt: To be arranged by the Developer / Concessionaire

    - Expenses

Expenses can broadly be classified based on the phases in which they are incurred, viz. construction period expenses and operation & maintenance period expenses.

    - Construction Period Expenses

- Preliminary and pre-operative expenses

- Contingency allowance

- Interest during construction period

- Finance Charges

    - Operation and Maintenance Period Expenses

- Toll collection expenses

- Administrative expenses for day-to-day operation including insurances

- Maintenance expenses, which include routine and periodic maintenance

- Interest expenses incurred for servicing term loans

- Tax

    - Financial Viability

To assess whether the project is a profitable proposition, the returns to investors are measured by the post-tax project FIRR and the equity FIRR, which is estimated from the cash-flow statements, based on discounted cash-flow technique. The returns expected by the investors are a function of the value of equity issues on the Indian stock Markets, Interest rates on commercial loans, the risk profile of the investment and alternative investment opportunities. To qualify the project in terms of attractive financial returns, the following criteria are adopted:

- Post tax IRR on Project Investment	:	minimum 12%

- Post tax IRR on Equity			:	minimum 12%

- DSCR					:	>1.0

- BCR					:	>1.3

- NPV @ 12%				:	must be positive

  - Recommendation & Conclusion on Type of Financing

- Project road section is financially not viable based on the forecasted traffic and mort&H user fee with 40% government subsidy and maximum concession   period of 15 years

- Therefore under epc contract option proposed for the entire project section with single package and 15 YEARS concession period is adopted.