## Improvement Proposals and Design

## General

This chapter describes the various improvement proposals and their necessities to upgrade the existing carriageway facility of project road into two lane with paved shoulder in accordance to the Indian standard configuration and design standards proposed for the project road. These improvement proposals are based on the findings of various engineering features carried out on the project roads such as Traffic Survey and Analysis (Chapter-5), Inventory Data and Pavement Investigations (Chapter-4).

The improvement proposals for proposed widening include the provisions for the following major items:

- Curvature Improvement

- Realignment

- Widening Proposal

- Proposed Pavement Design

- Bridge and Cross Drainage Structures

- Traffic Control and Safety Measures

## Design Standards

## Summary

Following is a summary of the recommended design standards proposed to be adopted for the project road other than service road and intersections:

|  | Design Speed (Km/hr) |  |  |
| --- | --- | --- | --- |
|  | Mountainous Terrain | : | 60 (Ruling), 40(Minimum) |
|  | Plain Terrain |  | 100 (Ruling), 80(Minimum) |
|  | Level of Service | : | B |
|  | Roadway Widths (m) Mountainous Terrain Plain Terrain | : | 11 m for 2-lanes with paved shoulders/ Granular Shoulder 14m for 2-lanes with paved shoulders/ Granular Shoulder |
|  | Roadway Elements Mountainous Terrain |  |  |
|  | With Retaining wall and parapet Without Retaining wall Built up Area with retaining wall Plain Terrain | : | Carriageway - 2-lane- 2X3.5m Paved Shoulder - 2-lane with PSS- 2x1.5m Carriageway - 2-lane- 2X3.5m Paved Shoulder - 2-lane with PSS- 2x1.5m Unpaved Shoulder - 2-lane with - 1.0m Carriageway - 2-lane- 2X3.5m Paved Shoulder - 2-lane with PSS- 2x1.5m Carriageway - 2-lane- 2X3.5m Paved Shoulder - 2-lane with PSS- 2x1.5m Unpaved Shoulder - 2- lane -2X2.0m |
|  | Camber |  | Carriageway Flexible- 2.50% Rigid - 2.00 % Paved Shoulder Flexible- 2.50% Rigid - 2.00 % Unpaved Shoulder Flexible- 3.50% Rigid - 3.00 % |
|  | Right of Way |  | As per Plan and Profile |
|  | Embankment/ Cutting Slope Fill height, up to 3.0 m Fill height from 3.0 m to 6.0 m Fill height exceeding 6.0 m |  | In filling- 1V: 2 H In filling- 1V: 1.5 H To be designed based on soil parameters, (IRC: 75) In cutting- 1V:1H |
|  | Stopping Sight Distance Intermediate sight distance |  | 20 m for design speed of 20 km/hr 25 m for design speed of 25 km/hr 30 m for design speed of 30 km/hr 40 m for design speed of 35 Km /hr 45m for design speed of 40km/hr 60 m for design speed of 50km/hr 90 m for design speed of 60km/hr 120m for design speed of 80 km/hr 180m for design speed of 100km/hr 40 m for design speed of 20 km/hr 50 m for design speed of 25 km/hr 60 m for design speed of 30 km/hr 80 m for design speed of 35 Km /hr 90 m for design speed of 40km/hr 120 m for design speed of 50km/hr 180 m for design speed of 60km/hr 240m for design speed of 80km/hr 360m for design speed of 100km/hr |
|  | Super-elevation Mountainous Terrain Plain Terrain |  | Maximum 10% Maximum 5% |
|  | Radii for Horizontal Curves Mountainous Terrain Plain Terrain |  | Ruling Minimum 150 m Absolute minimum 75 m Ruling Minimum 400 m Absolute minimum 250 m |
|  | Ruling Gradient Mountainous Terrain Plain Terrain |  | 5.0% 2.5% |
|  | Minimum k factor Summit Curve Mountainous Terrain Plain Terrain |  | Desirable: 8 Minimum: 5 Desirable: 74 Minimum: 38 |
|  | Valley Curve Mountainous Terrain Plain Terrain |  | Desirable: 10 Minimum: 7 Desirable: 42 Minimum: 28 |
|  | Bridge Clearance |  |  |
|  | Vehicular underpass |  | 5.5 m |
|  | Cattle and Pedestrian |  | 3.0m |
|  | Design Flood Frequency |  |  |
|  | Bridges |  | 100 years |
|  | Sewers and Ditches |  | 60 years |

## Road Functional Classification

National Highways & Infrastructure Development Corporation Ltd., Ministry of Road Transport & Highways, Government of India, has been assigned the work of preparation of feasibility study / DPR of road stretches/ corridors for up-gradation to two/ four laning with paved shoulder NH Configuration.

In pursuance of the above, have been appointed as Consultants to carry consultancy services for Project Management including preparation of Detailed Project Report for Construction of 2 Lane with Paved Shoulder Chenani – Sudhmahadev section of NH-244 (Old NH-1B)-Phase-I in the state of Jammu and Kashmir.

## Geometric Design

## General

Geometric design of a highway is the process whereby the layout of the road in specific terrain is designed to meet the needs of the road users keeping in view the road function, type and volume of traffic, potential traffic hazards and safety as well as convenience of the road users. The principal areas of control for fulfilment of this objective are- the horizontal alignment, vertical alignment and the road cross-section.

The Consultants have referred to the latest IRC publications and MORT&H circulars regarding design standards for National Highways in India. After careful review of all available data and requirements of the project road the proposed Design Standards for adoption on the project road have been recommended.

## Design Speed

The project road passes through mountainous and hilly terrain. For geometric design of the highway, design speed is used as an index which links road function, traffic flow and terrain.  An appropriate design speed should correspond to general topography and adjacent land use. The speed selected for design should also cater to travel needs and behaviour of the road users.

The ruling design speed corresponding to the type of terrain as per IRC: SP: 73-2015, AASHTO 2004 and TAC (1999) are as follows:

- Table 8.1: Design Speed Standards

| Terrain Classification | Design Speed (km/h) |  |
| --- | --- | --- |
|  | Desirable | Minimum |
| Plain &Rolling | 100 | 80 |
| Mountainous & Steep | 60 | 40 |

## Levels of Service (LOS)

The Level of Service (LOS) characterizes the operating conditions on the roadway in terms of traffic performance measures related to speed and travel time, freedom to manoeuvre, traffic interruptions, and comfort and convenience. The levels of service range from level-of-service A (least congested) to level-of-service F (most congested). The Highways Capacity Manual (HCM) provides the following levels of service definitions:

- Table 8.2: Standards for Level of Service

| Level of Service (LOS) | General Operating Conditions |
| --- | --- |
| A | Free flow |
| B | Reasonably free flow |
| C | Stable flow |
| D | Approaching unstable flow |
| E | Unstable flow |
| F | Forced or breakdown flow |

Considering the importance of the highway, whereas Level of Service (LOS) ‘B’ is desirable and level of service up to LOS-‘C’ may be acceptable.

## Cross Sectional Elements

Adequate roadway width will be provided for the requisite number of traffic lanes besides the shoulders .

As specified in the IRC: 73-2015 in general, standard lane width shall be 3.5m for project highway. Based on a comparative review of international standards and safety, the values proposed to be adopted for the roadway elements by the Consultants for the project highway are as follows:

- Roadway Width for Two Lane Highways

- Table 8.3A: Road Cross Section

| Item | Two-Lane with Paved Shoulder |
| --- | --- |
| Carriageways | 2X3.5m |
| Paved shoulder | 2X1.5 m |
| Unpaved shoulder mountainous terrain (if without retaining wall) | 1X1m |
| Total Roadway width mountainous terrain | 11 m |

| Item | Two-Lane with Paved Shoulder |
| --- | --- |
| Carriageways | 2X3.5m |
| Paved shoulder | 2X1.5 m |
| Unpaved shoulder Plain terrain | 2X2m |
| Total Roadway width Plain terrain | 14m |

- Lane Width

Lane width has a significant influence on the safety and comfort of the road. The capacity of a roadway is marked by affected by the lane width. In general, safety increases with wider lanes up to a width of about 3.7 m. The lane width as per IRC: SP 73: 2015 is 3.5 m.

Experience shows that operating speed normally remains less than the design speed because of the partially access controlled facility and the other ambient conditions. Based on this assumption a 3.5 m lane width is proposed. This also concurs with other National Highways in India currently under construction.

- Shoulders

Shoulders are a critical element of the roadway cross section. Shoulders provide recovery area for errant vehicles; a refuge for stopped or disabled vehicles; and access for emergency and maintenance vehicles. Shoulders can also provide an opportunity to improve sight distance through cut sections.

According to IRC: SP 73-2015 for two lane highways the normal shoulder width shall be 1.5 m paved shoulder on either side for mountainous terrain.

- Pavement Camber (Cross fall)

IRC: SP 73 2015 recommends the following camber for various surface types:

- Table 8.4: Provision for Cross Fall

| Category of surface | Annual Low rainfall (less than 1500 mm) | Annual High rainfall (more than 1500 mm) |
| --- | --- | --- |
| Bituminous | 2.5% | 2.5% |
| Cement Concrete | 2.0% | 2.0% |
| Metal/Gravel | 2.5% | 3.0% |
| Earth | 3.0% | 4.0% |

Considering of bituminous surfacing (bituminous concrete) the Consultants propose to provide a camber of 2.5 % for the main carriageway as well as paved shoulders and 3.5 % for the unpaved shoulder.

- Land Width (Right of Way)

The IRC: SP 73-2015 specifies that Two laning shall be accommodated within the existing ROW to the extent possible. However, additional land, if required for accommodating the two laning cross sections, improvement of geometrics, realignment, junctions, bypasses etc., shall be acquired by the Authority.

The Consultants therefore, propose 24 m ROW in mountainous terrains for two lane sections.

- Embankment Slopes

The slope of embankment is linked with its height. In accordance with the Manual for Safety in Road Design (MoRT&H publication), the following are proposed to be adopted:

| Ht of embankment 4.5 m and above | 2 H : 1V with crash barriers |
| --- | --- |
| Ht of embankment 3 m to 4.5 m | 2.5 H : 1 V |
| Ht of embankment 1.5 m to 3 m | 3 H : 1 V |
| Ht of embankment less than 1.5 m | 4 H : 1 V |

As per IRC: SP: 73-2015 the side slopes for embankment shall not be steeper than 2H:1V unless soil is retained by suitable soil retaining structure. The side slopes of cutting shall be provided in accordance with the nature of soil encountered. The slope shall be stable for type of strata. Where required, benching including use of slope stability measures like pitching, breast wall, etc. shall be adopted to make the slopes stable and safe.

The Consultants propose to provide slopes of 2H:1V in Fill sections. Cut slopes are proposed as 1H: 1V in general however, these sections will be specifically analysed for stability before adopting this slope or steeper slopes.

## Horizontal Alignment

- General

For balance in highway design, all geometrical elements should be determined for consistent operation under the design speed in general. A horizontal alignment should be as smooth and consistent as possible with the surrounding topography. To achieve that, an appropriate blending with the natural contours is preferable to the one with long tangents through the terrain.

- Sight Distances

Visibility is an important requirement for the safety of travel on roads. For this it is necessary that sight distance of adequate length is available in different situations, to permit drivers enough time and distance to control their vehicles so that chances of accidents are minimized. Sight distance is a direct function of the design speed.  On divided highways, the design should correspond to Stopping Sight Distance, which is the clear distance ahead needed by a driver to bring his vehicle to a stop before meeting a stationary object in his path. On two-lane roads, normally intermediate sight distance should be available throughout for design purposed. In stretches where even intermediate sight distance is not available, safe stopping site distance should be provided with traffic signs depicting “Overhead Prohibited” at all such locations.

Sight distance corresponding to various design speeds are given below.

- Table 8.6A: Sight distance for various Speeds

| Design Speed Km/h | IRC SP 73:2015 |  |
| --- | --- | --- |
|  | Stopping Sight Distance (m) | Intermediate Sight Distance (m) |
| 20 | 20 | 40 |
| 25 | 25 | 50 |
| 30 | 30 | 60 |
| 35 | 40 | 80 |
| 40 | 45 | 90 |
| 50 | 60 | 120 |
| 60 | 90 | 180 |
| 80 | 120 | 240 |
| 100 | 180 | 360 |

It is desirable to design the highway for more liberal values for operational convenience.  An appropriate allowance would be considered to take care of the effect of adverse incidents.  The value recommended by IRC & guidelines are proposed to be adopted in design.

- Horizontal Curve

The minimum horizontal curve radius is the limiting values of curvature for a given design speed and is determined based on from the maximum rate of super elevation and the side friction factor. As per the IRC: SP:73 – 2015 the minimum ruling radii of Horizontal curve for National Highways corresponding to different terrain conditions are as follows:

- Table 8.7: Horizontal Radii Criteria

| Type of Terrain | Minimum Radii of Horizontal Curve |  |
| --- | --- | --- |
|  | Desirable Minimum | Absolute Minimum |
| Mountainous | 150 | 75 |
| Plain | 400 | 250 |

Absolute minimum and ruling minimum radii correspond to the minimum design speed and the ruling design speeds respectively.

On new roads, horizontal curves are designed with liberal radius provision that blends well the overall geometry and topography.  However, for locations with constraints and to make use of available roadway, it is proposed to keep minimum radius in accordance with the IRC recommendations. The horizontal curve detail is given in Annexure 8.1.

- Table 8.8: Adopted Horizontal Radii

| Speed (km/h) | Absolute Minimum Radius (m) |
| --- | --- |
| 100 | 400 |
| 80 | 250 |
| 65 | 155 |
| 50 | 90 |

- Transition (Spiral) Curves

The purpose of a transition (spiral) curve is to provide a smooth and aesthetically pleasing transition from a tangent and a circular curve. In addition, the transition curves provide the necessary length for attainment of super-elevation runoff.

The IRC: 73-2015 and IRC :38-1988 design standards suggest 130 m, 90 m,80m and 75 m transition curve lengths for circular curves of radii 400 m, 250 (design speeds of 100 km/hr, 80 km/). The AASHTO (2001) design guidelines specify transition curve lengths of 72 m, 65 m and 50 m; and the TAC (1999) design guidelines recommend transition curve lengths of 80 m, 80 m and 50 m for curve radii of 440 m, 250 m, 90 m (design speeds of 100 km/hr, 80 km/hr and 50 km/hr) respectively.

It is proposed to adopt transition curve lengths of 55 m, 25 m, 15 m and 15 m for design speeds of 50km/hr, 40 km/hr, 30 km/hr and 25 km/hr respectively at their minimum recommended moves.

- Extra Width of Pavement and Roadways

Since the project road is of two lane categories extra widening is necessary on curves having radius less than 300 m to counter balance mechanical and psychological disorder of the vehicle. Extra widening is achieved by increasing the width at a uniform rate along the curve. On curve having no transition, widening is achieved in same way as super elevation i.e. two third is being attained on the straight section before start of the curve and one third on the curve. In hill roads and on curves without transitions extra widening is provided on inner side of the curve. As per IRC: SP: 73-2015, the extra widening shall be increased as follows:

- Table 8.9: Extra width of Pavement and Roadway

| Radius of Curve | Extra Width |
| --- | --- |
| 75-100m | 0.9m |
| 101-300m | 0.6m |

The value and guide lines recommended by IRC are proposed to be adopted in design.

- Super-elevation

The limiting value of the super-elevation on the project road in both mountainous terrain is proposed to be 10% as per IRC: 48 Hill road Manual.

## Service Road Standards

There is No Service Road in the Project stretch.

## Vertical Alignment

- General

The vertical alignment should produce a smooth longitudinal profile consistent with standard of the road and of the terrain. Wherever possible horizontal and vertical curvature should be so combined that the safety and operational efficiency of the road is enhanced.

- Gradients

The IRC: SP: 73-2015 propose ruling vertical grades of 5.0 % for mountainous terrains; However, for the project road, the following standard is proposed.

- Table 8.10:  Vertical Gradient

| Terrain | Ruling (%) | Limiting (%) |
| --- | --- | --- |
| Mountainous | 5.0 | 6.0 |
| Plain | 2.5 | 3.3 |

- Vertical Curves

As per IRC: 73-2015 design standards, the minimum lengths of vertical curves are 60 m and 50 m for design speeds of 100 km/h and 80 km/h respectively and are 40 m and 30 m for design speeds of 60 km/h and 40 km /h respectively. At complex locations such as interchanges and major intersections the minimum lengths of vertical curves should be designed for safe decision sight distance.  The length of a vertical curve is calculated using the following equation:

L = K x A,

Where		L = Length of vertical curve in metres;

K = Coefficient, a measure of the flatness of a vertical curve; and

A = Algebraic difference of grade lines (%)

Summit or Crest Curves

According to AASHTO (2001) design guidelines, the minimum K values for stopping sight distance requirements are 52, 26 and 7 for design speeds of 100 km/hr, 80 km/h and 50 km/hr respectively.

According to TAC (1999) design guidelines, the minimum K valves for stopping sight distance requirements are 45 to 80, 24 to 36 and 6 to 16 for design speeds of 100 km/hr, 80 km/hr and 50 km/hr respectively.

The Consultants propose minimum summit curve K values of 75, 35, 20 and 15 for design speeds of 100 km/hr, 80 km/hr, 65km/hr and 50 km/hr respectively.

Valley or Sag Curves

The minimum K values for valley or sag curves, in accordance with AASHTO (2001) design guidelines are 45, 30 and 13 for design speeds of 100 km/hr, 80 km/hr and 50 km/hr respectively. The minimum K values for valley or sag curves, in accordance with TAC (1999) design guidelines are 37 to 50, 25 to 32 and 7 to 16 for design speeds of 100 km/hr, 80 km/hr and 50 km/hr respectively.

The Consultants propose minimum sag curve K values of 42, 30, 20 and 15 for design speeds of 100 km/hr, 80 km/hr, 65km/hr and 50 km/hr respectively.

| Terrain Categories | K -Value of Summit Curves |  | K- Value of Valley Curves |  | Minimum Length of curve |
| --- | --- | --- | --- | --- | --- |
|  | Desirable | Minimum | Desirable | Minimum | (m) |
| Plain | 74 | 38 | 42 | 28 | 60 |
| Rolling | 38 | 18 | 28 | 18 | 50 |
| Mountainous | 8 | 5 | 10 | 7 | 30 |

## Design Standards for Structures

The design of new structures shall be based on the following materials and loading-

## Materials

Concrete Grade

The minimum Grade of concrete in various elements shall be as under for moderate conditions of exposure:

Major Bridge				Minor Bridge/Culvert

All PCC		M 25						M 15

All RCC		M 30						M 25

All PSC		M 35						   -

Reinforcement Steel

-	High yield strength deformed bar/TMT shall be of grade Fe-415/Fe-500

-	Mild steel bar shall be of grade Fe-240

## Seismic Zone

The project road is in a seismic zone IV. It is proposed to design the bridges for seismic forces as mentioned in modified clause 222 of IRC: 6-2000.

## Pre-Stressing System

Following pre -stressing system may be adopted as a general system.

a) System (Post tensioning)		:	12T13/19T13 multi pull strand system of

“Freyssinet” or “ISMALCCL” or equivalent

b) Cables (Post tensioning)		:	12T13/19T13 with strands of 12.7mm

nominal dia.

c) High Tensile Steel

(for both post/pre-tensioning)

Strands	:	Nominal 12.7 mm dia 7 ply low relaxation Strands conforming to class 2 of IS:14268-95

Area	:	98.7 sq.mm per strand (nominal cross section area)

Ultimate load	:	183.71 KN per strand

Modulus of Elasticity			:	1.95E05 Mpa

d) Sheathing (Post tensioning)	:	75mmOD/90mmOD Bright metal corrugated flexible sheathing for 12T13/19T13 cables respectively.

e) Friction Coefficient (Post tensioning)		0.25/radian

f) Wobble Coefficient (Post tensioning)		0.0046/m

g) Anchorage Slip (Post tensioning)		6mm average

h) Loss of force due to relaxation	:	2.5% at 0.7 UTS after 1000 hrs. The final relaxation Values for design shall be 3.0 times the 1000hr. value as per cl 11.4 of IRC: 18-1985.

## Structural Steel

Composite construction consisting of structural steel girders with cast-in-situ deck slab may be proposed over deep valleys by keeping in view the seismic zone of the project roads. Superstructure weight shall be substantially reduced by using structural steel girders. Structural steel shall conform to IS: 226.

## Bearings

Reinforced elastomeric bearings shall be proposed for short span simply supported superstructures. Elastomeric bearings shall be designed as per IRC: 83 (Part II & III) and shall conform to Cl.2005 of MOSRT&H Specifications for Road & Bridges Works (4th Revision). RCC solid slab superstructures of culverts and minor bridges shall directly rest on pier/abutment caps with a tar paper in bearing.

Pot fixed/Pot PTFE sliding/ metallic bearings shall be proposed for long span simply supported superstructures and continuous superstructures. The loads and forces on the bearings shall be calculated to enable the manufacturer to design these bearings and these shall conform to Cl. 2006 of MOSR&TH Specifications for Road & Bridges Works (4th Revision).

## Expansion Joints

The following types of Expansion Joints shall be adopted:

Filler type expansion joints shall be proposed for minor bridges with solid slab superstructures having span lengths not exceeding 10 meters. These types of joints shall conform to Cl. 2605 of MOST’s Specifications for Road & Bridge Works (4th Revision).

Single Strip seal expansion joints shall be proposed for superstructures having movements up 80mm. (± 40mm).

The strip seal joints shall conform to Cl. 2607 of MOST’s Specification for Road and ~Bridges works (4th Revision).

Concrete Clear Covers:

For all reinforcement	                      -             As per IRC: 112 and 78

For other covers and inter duct spacing -	           As per IRC: 112

## Loads

Dead Loads

Following unit weights shall be assumed in the design as per IRC Codes.

Pre-stressed Concrete		:	2.5t/m3

Reinforced Concrete		:	2.5t/m3

Plain Cement Concrete		:	2.5 t/m3

Structural Steel			:	7.85t/m3

Dry Density of Soil		:	2.0t/m3

Saturated Density of Soil	:	2.2t/m3

Superimposed Dead Loads

Wearing coat	:	40mm thick Bituminous Concrete with total weight of 2.2 t/m3

Carriage way Live Load

Bridge Live load			   :  One lane of IRC  70R

2 Lane of Class A.

Whichever produces worst effect

The impact factor shall be as per Cl. 211 of IRC:6 for the relevant load combinations. The impact factor shall be as per Cl. 211 of IRC:6 for the relevant load combinations.

Longitudinal Forces

The following effects shall be considered for calculating the longitudinal forces in the design.

Braking forces as per the provision of Cl. 214 of IRC: 6-2000

Frictional resistance offered to the movement of free bearings due to change of temperature.

Distribution of longitudinal forces due to horizontal deformation of bearings/frictional resistance shall be carried out as per Cl. 214.5 of IRC:6 by assuming stiff supports.

Centrifugal Forces

Bridges on a horizontal curve shall be designed for centrifugal forces based on the following equation-

C = W*V2/127R,

Where C = Centrifugal force acting normal to the traffic.

W = Carriageway Live Load

V = Design speed of the Vehicles using the bridge in km per hour.

R = Radius of curvature in metres.

The centrifugal force shall be considered to act at 1.20m above the formation level of the bridge in the transverse direction. No impact value on carriageway live load shall be considered for calculating the centrifugal force.

Water Current Forces

The effect of water current forces shall be calculated in accordance with clause number 213 of IRC: 6-2000 on sub structure and foundations. High flood level and Velocity shall be calculated based on the details received from relevant Government departments or Local inquirie

Impact Forces

All the sub- structure and foundations in the river shall be designed for the impact due to striking of rolling boulders on the sub-structure in mountainous terrain. The magnitude of force shall be decided based on field studies and in consultation with client.

Earth Pressure

Horizontal forces due to earth pressure shall be calculated as per the provision of Cl. 217 of IRC:6 assuming the following soil properties:

Type of soil assumed for backfilling	: Dry Density of 1.8 t/cu.m and Submerged Density of 1.0t/cu.m

Angle of Internal friction			: ø = 30 degree

Angle of Wall Friction			: δ =20 degree

Coefficient of Friction µ at base 	:	tan(2/3ø), where ø is the angle of internal friction of substrata immediately under the foundation.

Live Load surcharge shall be considered as equivalent to 1.2m height of earth fill in case of abutments and equivalent to 0.6m height of earth fill in case of return/wing walls.

Wind Forces

Structures shall be designed for wind effects as stipulated as Cl. 212 of the IRC:6. The Wind force shall be considered in the following two ways. The design shall be governed by the one producing the worst effect.

- Full wind forces at right angle to the superstructure

- 65% of wind force as calculated in (i) above acting perpendicular to the superstructure and 35% acting in the traffic direction.

## Seismic Effect

The project road falls under seismic zone IV. Horizontal seismic force shall be calculated using the following formula-

Feq 	= 	Ah X (Dead Load + Appropriate Live Load)

Where, Ah = 	Horizontal seismic co-efficient = (Z/2) X (Sa/g)/(R/I)

Z 	= 	Zone factor

I 	= 	Important factor and is taken as 1.5 for important Bridges.

R 	= 	Response reduction factor and is equal to 2.5

Sa/g 	= 	Average response acceleration coefficient depending upon fundamental period of vibration T

T 	= 	Fundamental period of Bridge in seconds in horizontal vibrations.

The vertical seismic coefficient shall be considered in the case of structures built in seismic IV. The vertical seismic coefficient shall be considered as half of the horizontal seismic force.  Both horizontal and vertical seismic forces shall be assumed to act simultaneously for the design of bridge components.

## Temperature Range

The bridge structure/components i. e bearings and expansion joints shall be designed for a temperature variation of +-25 degree centigrade considering extreme climate.

The super structure shall be designed for effects of distribution of temperature across the deck depth as per stipulations of BD 37/88 suitably modified for the surfacing thickness.

## Differential Shrinkage Effects

A minimum reinforcement of 0.2% of cross sectional area in the longitudinal direction of the cast-in-situ slab shall be provided to cater for differential shrinkage stresses in superstructures with in-situ slab over pre-cast girders as per Cl.605.2 of IRC: 22-1986.

However, effects due to different shrinkage and/or different creep shall be duly accounted for in the design.

## Differential Settlement Effects

Differential Settlement effects for continuous superstructure units shall be appropriately assessed for each structure. However, in any case of differential settlement of ± 12 mm shall be accounted for in the design.

The differential settlement effects in continuous superstructures shall be accounted for under following conditions:

- A minimum of 12mm differential settlement of supports with half value of ‘E’.

- To simulate the bearing replacement conditions, a 12mm differential uplift with full value of ‘E’ shall be considered but without any live load on the superstructure.

## Buoyancy

100% buoyancy shall be considered while checking stability of foundations irrespective of their resting on soil/weathered rock/or hard rock. However, maximum base pressure shall also be checked under an additional condition with 50% buoyancy in cases where foundations are embedded into hard rock. Pore pressure uplift limited to 15% shall be considered while checking stresses of the substructure elements.

In the design of abutment, the effects of buoyancy shall be considered assuming the fill behind abutment has been removed by scour.

## Load Combination

All members shall be designed to safely sustain the most critical combination of various loads and forces that can coexist. Various load combinations as relevant with increase in permissible stresses considered in the design shall be as per Cl. 202 of IRC:6 and Cl.706 of IRC:78.

In addition, the stability of a bridge resting on neoprene/pot bearings shall be checked under one span dislodged condition. The load case shall be checked with seismic/wind load combinations.

## Design Criteria of Culverts

The culverts shall be designed as per relevant IRC codes and special publications. The following IRC codes have been adopted for design of culverts:

IRC : 5-2015 		General Features of Design;

IRC : 6-2014		Loads & Stresses;

IRC : 48  1998             Hill Road Manual

IRC : 40-2002 		Brick, Stone & Block Masonry;

IRC:SP:13 2004	Guidelines for the Design of Small Bridges and Culverts;

## Codes to be adopted for Design

Various codes of practices which shall be used for the design of culverts and bridges are mentioned below:

IRC Standards

IRC:5-2015 : Standard Specifications and Code of Practice for Road Bridges,

Section I- General Features of Design (Seventh Revision)

IRC:6-2014 : Standard Specifications and Code of Practice for Road Bridges,

Section II- Loads and Stresses (Fourth Revision)

IRC:7-1971 : Recommended Practice for Numbering Bridges and Culverts (First   Revision)

IRC: 112-2011: Design Criteria for Prestressed Concrete Road bridges ( Post -Tensioned Concrete)      (Third Revision)

IRC:112-2011: Standard Specifications and Code of Practice for Road Bridges

Section III – Cement Concrete (Plain and Reinforced)( Third Revision)

IRC:22-2015: Standard Specifications and Code of Practice for Road Bridges

Section VI – Composite Construction (Limit States Design)( Second Revision)

IRC:24-2010: Standard Specifications and Code of Practice for Road Bridges

Section IV – Brick, Stone and Block Masonry (Second Revision)

IRC:40-2002: Standard Specifications and Code of Practice for Road Bridges

Section V – Steel Road Bridges (Second Revision)

- IRC: 45-1972: Recommendations for Estimating the Resistance of Soil Below the maximum scour Level in the Design of Well Foundations of Bridges.

- IRC:54-1974: Lateral and Vertical Clearances at Underpasses for Vehicular Traffic

- IRC:78-2014 : Standard Specifications and Code of Practice for Road Bridges

Section VII – Foundation and Substructure (Second Revision)

- IRC: SP: 33-1989: Guidelines on Supplemental Measures for Design, Detailing & Durability of Important Bridge Structures.

- IRC:SP:48-1998 : Hill Road Manual

- IRC:83-2015 : Standard Specifications and Code of Practice for Road Bridges

- Section IX – Bearings, Part I: Metallic Bearings (First Revision)

- IRC:83-2015 : Standard Specifications and Code of Practice for Road Bridges

- Section IX – Bearings, Part II: Elastomeric Bearings

- IRC:83-2002 : Standard Specifications and Code of Practice for Road Bridges

- Section IX – Bearings, Part III: POT,POT-CUM-PTFE,PIN and Metallic Guide Bearings

- IRC:89-1997: Guidelines for Design and Construction of River Training & Control Works for Road Bridges (First Revision)

- IS:456-2000: Plain and Reinforced Concrete ( Fourth Revision)

- IS:2502-1963

- BS 5400-PartIX (For design of POT/POT-PTFE Bearings)

- IRC:SP: 13-2004 : Guidelines for the Design of  Small Bridges and Culverts( First Revision)

- IRC:SP: 35-1990: Guidelines for Inspection and Maintenance  of Bridges.

- IRC:SP: 40-1993: Guidelines on Techniques for Strengthening and Rehabilitation of Bridges

- IRC:SP: 47-1998 : Guidelines on Quality Systems for Road Bridges ( Plain, Reinforced, Prestressed and Composite Concrete)

- IRC: SP:51-2015 : -Guidelines for Load Testing of Bridges.

- IS: 808

- IS 2062/8500

MOSRT&H Specifications

- The specifications for road and bridges works of Ministry of Road Transport & Highways (latest editions) published by Indian Road congress shall be used for materials to be used for construction of bridge.

- MORT&H standards plans for single, double and triple cell box culverts with and without earth cushion.

- Standard Drawings for Road Bridges, RCC Solid Slab Superstructure (22.5 skew) for  spans 4m to 10m

Indian Standards

- IS:456-2000: Plain and Reinforced Concrete (Fourth Revision)

- IS:2502-1963

- IS: 808

- IS: 2062/8500

## Widening Scheme

To meet future traffic requirement, the existing carriageway is proposed to upgrade to achieve high speed of travel with comfort and safety. Concentric widening scheme is followed to minimise land acquisition issues and to ensure maximum utilisation of existing carriageway.

## Typical Cross-sections

Proposed cross-sections along with widening schedule is shown in Table 8.11 below and (Annexure 8.5).

Table 8.11: Proposed Improvement Proposal

| Sr. No. | Design Chainage |  | Length (Km) | TCS Type | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | From (Km) | To (Km) |  |  |  |
| 1 | 0.000 | 0.800 | 0.800 | 3 | One side Valley & One side Built up or open area |
| 2 | 0.800 | 0.840 | 0.040 | 6 | Realignment |
| 3 | 0.840 | 0.890 | 0.050 | 4 | Approach to Bridge |
| 4 | 0.890 | 0.965 | 0.075 | Bridge | Bridge |
| 5 | 0.965 | 1.015 | 0.050 | 4 | Approach to Bridge |
| 6 | 1.015 | 1.274 | 0.259 | 2 | One side Hill & One side Valley (Drain on Hill Side) |
| 7 | 1.274 | 1.466 | 0.192 | 2 | One side Hill & One side Valley (Drain on Hill Side) |
| 8 | 1.466 | 2.400 | 0.934 | 6 | Realignment |
| 9 | 2.400 | 2.450 | 0.050 | 4 | Approach to Bridge |
| 10 | 2.450 | 2.700 | 0.250 | Bridge | Bridge |
| 11 | 2.700 | 2.750 | 0.050 | 4 | Approach to Bridge |
| 12 | 2.750 | 3.050 | 0.300 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 13 | 3.050 | 3.300 | 0.250 | 6 | Realignment |
| 14 | 3.300 | 4.100 | 0.800 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 15 | 4.100 | 4.167 | 0.067 | 6 | Realignment |
| 16 | 4.167 | 4.217 | 0.050 | 4 | Approach to Bridge |
| 17 | 4.217 | 4.223 | 0.006 | Bridge | Bridge |
| 18 | 4.223 | 4.273 | 0.050 | 4 | Approach to Bridge |
| 19 | 4.273 | 4.900 | 0.627 | 6 | Realignment |
| 20 | 4.900 | 5.400 | 0.500 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 21 | 5.400 | 5.558 | 0.158 | 6 | Realignment |
| 22 | 5.558 | 5.700 | 0.142 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 23 | 5.700 | 6.000 | 0.300 | 6 | Realignment |
| 24 | 6.000 | 6.185 | 0.185 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 25 | 6.185 | 6.400 | 0.215 | 6 | Realignment |
| 26 | 6.400 | 6.600 | 0.200 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 27 | 6.600 | 6.825 | 0.225 | 6 | Realignment |
| 28 | 6.825 | 7.400 | 0.575 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 29 | 7.400 | 7.447 | 0.047 | 6 | Realignment |
| 30 | 7.447 | 7.497 | 0.050 | 4 | Approach to Bridge |
| 31 | 7.497 | 7.503 | 0.006 | Bridge | Bridge |
| 32 | 7.503 | 7.553 | 0.050 | 4 | Approach to Bridge |
| 33 | 7.553 | 7.700 | 0.147 | 6 | Realignment |
| 34 | 7.700 | 8.677 | 0.977 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 35 | 8.677 | 8.727 | 0.050 | 4 | Approach to Bridge |
| 36 | 8.727 | 8.733 | 0.006 | Bridge | Bridge |
| 37 | 8.733 | 8.783 | 0.050 | 4 | Approach to Bridge |
| 38 | 8.783 | 8.800 | 0.017 | 6 | Realignment |
| 39 | 8.800 | 8.897 | 0.097 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 40 | 8.897 | 8.947 | 0.050 | 4 | Approach to Bridge |
| 41 | 8.947 | 8.953 | 0.006 | Bridge | Bridge |
| 42 | 8.953 | 9.003 | 0.050 | 4 | Approach to Bridge |
| 43 | 9.003 | 9.254 | 0.251 | 6 | Realignment |
| 44 | 9.254 | 9.304 | 0.050 | 4 | Approach to Bridge |
| 45 | 9.304 | 9.364 | 0.060 | Bridge | Bridge |
| 46 | 9.364 | 9.414 | 0.050 | 4 | Approach to Bridge |
| 47 | 9.414 | 10.200 | 0.786 | 5 | One side Hill & One side Valley (Breast Wall and Gabion Box Protection on hill side) |
| 48 | 10.200 | 10.600 | 0.400 | 6 | Realignment |
| 49 | 10.600 | 10.900 | 0.300 | 5 | One side Hill & One side Valley (Breast Wall and Gabion Box Protection on hill side) |
| 50 | 10.900 | 11.100 | 0.200 | 6 | Realignment |
| 51 | 11.100 | 11.400 | 0.300 | 5 | One side Hill & One side Valley (Breast Wall and Gabion Box Protection on hill side) |
| 52 | 11.400 | 11.625 | 0.225 | 6 | Realignment |
| 53 | 11.625 | 11.700 | 0.075 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 54 | 11.700 | 12.100 | 0.400 | 6 | Realignment |
| 55 | 12.100 | 13.217 | 1.117 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 56 | 13.217 | 13.267 | 0.050 | 4 | Approach to Bridge |
| 57 | 13.267 | 13.273 | 0.006 | Bridge | Bridge |
| 58 | 13.273 | 13.323 | 0.050 | 4 | Approach to Bridge |
| 59 | 13.323 | 13.555 | 0.255 | 6 | Realignment |
| 60 | 13.555 | 14.180 | 0.625 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 61 | 14.180 | 14.447 | 0.267 | 6 | Realignment |
| 62 | 14.447 | 14.497 | 0.050 | 4 | Approach to Bridge |
| 63 | 14.497 | 14.503 | 0.006 | Bridge | Bridge |
| 64 | 14.503 | 14.553 | 0.050 | 4 | Approach to Bridge |
| 65 | 14.553 | 14.700 | 0.147 | 6 | Realignment |
| 66 | 14.700 | 14.787 | 0.087 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 67 | 14.787 | 14.837 | 0.050 | 4 | Approach to Bridge |
| 68 | 14.837 | 14.843 | 0.006 | Bridge | Bridge |
| 69 | 14.843 | 14.893 | 0.050 | 4 | Approach to Bridge |
| 70 | 14.893 | 15.055 | 0.162 | 6 | Realignment |
| 71 | 15.055 | 16.300 | 1.245 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 72 | 16.300 | 16.700 | 0.400 | 6 | Realignment |
| 73 | 16.700 | 16.990 | 0.290 | 5 | One side Hill & One side Valley (Breast Wall and Gabion Box Protection on hill side) |
|  | Total Length |  | 16.990 |  |  |

Details of TCS

| Summary of TCS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sr. No. | Detail | TCS | Length |  |
|  |  |  | (m) | (Km) |
| 1 | One side Hill & One side Valley (Breast Wall on hill side) | 1 | 6.902 | 6902 |
| 2 | One side Hill & One side Valley (Drain on Hill Side) | 2 | 0.451 | 451 |
| 3 | One side Valley & One side Built up or open area | 3 | 0.800 | 800 |
| 4 | Approach to Bridge | 4 | 1.000 | 1000 |
| 5 | One side Hill & One side Valley (Breast Wall and Gabion Box Protection on hill side) | 5 | 1.676 | 1676 |
| 6 | Realignment | 6 | 5.734 | 5734 |
| 7 | Bridge |  | 0.427 | 427 |
|  | Total Length |  | 16990.00 | 16.990 |

  - Requirement of Bypass

No bypass is being proposed along the project road.

## 8.5	Geometric Improvement Design

As per the IRC: SP: 73-2015 the project highway should be design with 60 km/hr ruling speed and minimum speed of 40 km/hr. Existing geometry of the project highway has been improved to achieve the minimum design speed of 60km/hr except with following locations which are proposed as “Speed Restriction Zone” and improved with design speed of 20km/hr to 30 km/hr to utilise existing bridges, bridges under construction, to reduce land acquisitions and R&R in the project.

## Improvement of Bridges

The following approach and methodology for the finalization of designs and drawings for the existing and proposed bridge structures are proposed.

## General

  - Review of Past records like Studies, Reports and Data’s.

  - Data relevant to bridges shall also be collected from the PWD (NH) and irrigation departments of Chhattisgarh. The following data will generally be looked to the extent available:

  - Hydrological and geo-technical reports of the existing CD structures.

  - Complete ‘as built’ drawings of existing two-lane bridges along with their design calculations, if available.

  - Details of repair/rehabilitation, if any, carried out for the existing Single lane bridges.

  - Nature and extent of damage observed during floods to any of the existing two-lane bridges.

  - Utility services to be carried over the bridges.

  - Any other engineering data found suitable for the detailed engineering of proposed bridge structures.

This Chapter covers the various methodologies and design criteria, Codal provisions for proposed Bridges.

Following are the grades of construction material proposed for the project

Foundation

Concrete Grade	: M35 for Bridges & Culverts

Reinforcement		:  HYSD of grade Fe500

Abutment / Abutment Cap and Pier / Pier Cap

Concrete Grade	: M35 for Bridges with RCC Substructure and Foundation

Reinforcement		:  HYSD of grade Fe500

Superstructure

Concrete Grade	:  M35 for RCC girders

:  M45 for PSC precast girders

:  M35 for Bridge decks over girders

:  M35 for RCC solid slabs

:  M35 for RCC solid slab of Slab Culverts

Reinforcement		: HYSD of grade Fe500

Structural Steel  	: Grade E250 (Fe410 W B grade) (For ROB)

Crash Barrier

Concrete Grade	: M40

Reinforcement		: HYSD steel of grade Fe500

Approach Slab

Concrete Grade	: M30

Reinforcement		: HYSD steel of grade Fe500

Clear Cover to any Reinforcement is followed as below

Foundation		: 75 mm

Substructure		: 50 mm

Superstructure		: 40 mm

Bearings

- For Span 6.00 - 10.00 m, Tar paper bearings shall be adopted for slab superstructure.

- For Span 10.00 – 20.00 m, Elastomeric Bearing for RCC solid slab, RCC girder superstructure.

- For Larger span, POT / PTFE bearing for RCC / PSC girder superstructure.

Expansion Joints

Compression seal for slab superstructure and strip seal for girder superstructure.

Wearing Coat

- Cross –drainage structure: 40 mm thick bituminous concrete overlaid with 16 mm thick mastic asphalt.

- Minor and Major Bridges: 40 mm thick bituminous concrete overlaid with 25 mm thick mastic asphalt.

Approaches

RCC Return or Retaining wall for Culverts and Bridges are considered.

Drainage Provisions.

Drainage spouts shall be placed not greater than 10. 00 m centre to centre. Down take pipes will be provided to dispose the water.

Margins in Material (FOS)

All critical sections shall be checked for stresses under various load combinations. A suitable margin (preferably 8-10%) shall be there between maximum stress and allowable stress in concrete as well as reinforcement in the final design.

Conceptual Guidelines for Structure

Following guidelines will be followed in design and construction of structures:

- The existing structures will be widened or extended to match the new road cross sections.

- For Major and Minor bridges in urban or rural areas, open median shall be provided with miimum 3.50 m clear gap between two crash barriers of bridges.

- New Bridges will be planned without affecting the foundations of adjacent existing bridges, if any.

- All new / reconstructed pipe culverts will constitute minimum 1.20 m diameter size pipes that confirm to NP4 specifications. The existing 0.90 m or more diameter pipe culverts will be extended to new carriageway with the same diameter or 1.20 m diameter pipes. In case where the culverts are hydraulically inadequate, shall be replaced by RCC Box / RCC Slab culvert of adequate size.

- Rehabilitation of substructure / superstructure of the existing Bridges which are proposed to be retained, including, but not limited to, replacement of bearings, expansion joints, pitching, bed protection, provision of crash barrier and railings, shall be done by the Concessionaire in accordance with - the Concession Agreement.

Relevant Codes Followed for Design of Structures

List of IRC Codes

The list of IRC codes for the design of various all types of structures are as follows.

- IRC: 5-2015 - Standard Specifications & code of Practice for Road Bridges. Section-I General features of Design (8th revision)

- IRC: 6-2014 - Standard Specifications & code of Practice for Road Bridges. Section-II Loads and Stresses (5th revision)

- IRC: 7-1971 - Recommended Practice for numbering Bridges and culverts (1st revision)

- IRC: 112-2011 - Standard Specifications & code of Practice for Road Bridges

- IRC: 24-2010 - Standard Specifications & code of Practice for Road Bridges. Section-V Steel Road bridges (1st revision)

- IRC: 78-2014 - Standard Specification & code of Practice for Road Bridges. Section-VII Foundations and Substructure (2nd revision)

- IRC: 83-2011 - Standard Specifications & code of Practice for Road Bridges. Section-IX Bearings Part II- Elastomeric Bearings

- IRC: 83-2002 - Standard Specifications & code of Practice for Road Bridges. Section-IX Bearings Part III- POT/PTFE, PIN and METALLIC GUIDED Bearings

- IRC: 89-2010 - Guidelines for Design & Construction of River training & Control works for Road Bridges (1st revision).

List of IRC-SP Codes

- IRC: SP: 13-2004 - Guidelines for the Design of Small Bridges and Culverts

- IRC: SP: 35-1990 - Inspection and maintenance of Bridges

- IRC: SP: 40-1993 - Guidelines on Strengthening and Rehabilitation of Bridges

- IRC: SP: 84-2014 – Manual of Specifications and Standards for Four Laning of Highways through public private partnership

Ministry of Surface Transport Publications

MORT&H Specifications for Road and Bridge Works, 2013 (Fifth Revision)

Existing structures on the project road have been classified in three categories based on the reconnaissance survey.

- Culverts

Structures having an overall length up to 6.0m shall be treated as culverts. Most of the culverts have no protection works.

- Minor Bridges

Structures having a length between inner face of dirt walls more than 6.0m and up to 60.0m shall be treated as minor bridges. These bridges on project roads are of reinforced concrete solid slab, structural steel trusses/ girder and RCC T- beam girders type. Minor bridges seen during the site visit have spans varying from 8.0m to 50.0 m with R&R masonry wall type abutments and stone masonry/plain cement concrete wall type piers. The protection works around abutments are either damaged or not existing.

- Major Bridges

Structures having a length of more than 60.0m shall be called major bridges.

## Proposed Re-construction of Existing Cross Drainage Structures

Project road mainly consists of bridges, culverts and causeways. Most of the culverts are in good condition and are not choked. The number of existing culverts, which could be retained, shall be known after finalization of culvert inventory and condition survey. The existing culverts, which are in good conditions but does not have sufficient / adequate width for 2-lane, shall be widened. Damaged causeways will be proposed for reconstruction.

## Proposed Re-construction of Existing Bridges

It is found based on the site visit most of the bridges are in fair to good condition but inadequate width. The detailed condition survey of the existing bridges is carried out to ascertain their conditions. The existing bridges with adequate carriageway width for a two-lane bridge will be retained with some rehabilitation work. At few Placing overtopping is observed and CD structures are proposed to be reconstructed.

## Type of Proposed Bridges

Following type of super-structures will be most suitable for bridges:

- Structural steel girders/trusses

- Reinforced concrete pre-cast bridges

- Pre-cast Post tensioned concrete bridges

- RCC Box type structures where SBC is less

Following type of sub-structures will be most suitable for bridges:

- RCC abutment and pier for bridges

- PCC abutment and pier for culverts

Piers shall be avoided in the mid-stream where velocity of water is more than 5.0m/second. It is generally seen that it is very difficult to construct sub-structure in such locations and there is possibility of bridge being washed away. Thus, all efforts shall be made to provide large spans for the mid-stream in order to avoid any pier.

Circular/cellular circular/wall type piers shall be used after considering the aesthetics and economy. Solid wall type abutments/counter fort type abutments based on the height shall be selected. Counter fort type abutments are generally provided if height of the abutments is more than 10.0 metres

Submersible Structures

Submersible Bridges and Causeway are highly suitable where the floods are flash and do not interrupt the traffic for long period.

These are normally built on non-erodible bed rock with protective pitching or apron.

Though submersible bridges are cheap compared to high level bridges, they need greater maintenance for approaches if there is considerable spread of water. Design of hand rails, impact of floating debris and the hydrodynamic effect of the water acting over the whole bridges also required to design submersible Bridges. These have to be considered along with the buoyancy in design.

## Improvement Proposal of Existing and Proposal of New CD Structure

There are 3 existing minor bridges in the project road. During inventory and Condition survey, it was found that few bridges are in poor condition and some bridges are insufficient width. Based on hydraulics, few bridges are overtopped and converted to high level bridges. The Brief detail of existing structures has been given different improvement proposals below table:

Summary of Structures

- Summary of structures proposed

| SL.NO. | DESCRIPTION | No. Of Structures | REMARKS |
| --- | --- | --- | --- |
| 1 | PIPE CULVERTS |  |  |
|  | Causeway -PQC |  |  |
| (i) | Retaining & Widening | 0 |  |
| (ii) | Dismantling & Reconstruction Pipe Culvert with Box Culvert | 2 |  |
| (iii) | New Proposals | 0 |  |
| (iv) | Abandoned | 1 |  |
|  | Pipe Culverts |  |  |
| (i) | Retaining & Widening | 0 |  |
| (ii) | Dismantling & Reconstruction Pipe Culvert with Box Culvert | 37 |  |
| (iii) | New Proposals | 0 |  |
| (iv) | Abandoned | 34 |  |
| 2 | SLAB CULVERTS |  |  |
| (i) | Retaining & Widening | 0 |  |
| (ii) | Dismantling & Reconstruction Slab Culvert with Box Culvert | 12 |  |
| (iii) | New Proposals | 0 |  |
| (iv) | Abandoned | 4 |  |
| 3 | BOX CULVERTS |  |  |
| (i) | Retaining & Widening | 0 |  |
| (ii) | Dismantling & Reconstruction | 0 |  |
| (iii) | New Proposals | 30 |  |
| (iv) | Abandoned | 0 |  |
| 4 | SLAB CULVERTS |  |  |
| (i) | Retaining & Widening | 0 |  |
| (ii) | Dismantling & Reconstruction | 8 |  |
| (iii) | New Proposals | 0 |  |
| (iv) | Abandoned | 4 |  |
| 4 | Blocked Culverts |  |  |
| (i) | Retaining & Widening | 0 |  |
| (ii) | Dismantling & Reconstruction | 1 |  |
| (iii) | New Proposals | 0 |  |
| (iv) | Abandoned | 1 |  |
| 5 | UNDERPASSES |  |  |
| (a) | Vehicular underpasses | 0 |  |
| (b) | Cattle underpasses | 0 |  |
| 6 | MINOR BRIDGES |  |  |
| (i) | Abandoned | 3 |  |
| (ii) | Dismantling & Reconstruction | 0 |  |
| (iii) | Retained | 0 |  |
| (iv) | New proposals | 8 |  |
| 7 | MAJOR BRIDGES |  |  |
| (i) | To be widened | 0 |  |
| (ii) | Dismantling & Reconstruction | 0 |  |
| (iii) | Retained | 0 |  |
| (iv) | New proposals | 2 |  |

# Detail of proposed Structures

Based on Hydraulics, alignment modifications, widening requirement etc., improvement of

existing C.D. structures as well as proposed new CD structures along the existing alignment is prepared and presented detailed vide Annexure 8.2 & 8.3.

| Sr. No. | Design Chainage of Bridges (km) | Span Length (km) | Alignment | Remarks |
| --- | --- | --- | --- | --- |
| 1 | 0.928 | 0.075 | New Alignment | Major Bridge |
| 2 | 2.575 | 0.250 | New Alignment | Major Bridge |
| 3 | 4.220 | 0.006 | New Alignment | Minor Bridge |
| 4 | 7.500 | 0.006 | New Alignment | Minor Bridge |
| 5 | 8.730 | 0.006 | New Alignment | Minor Bridge |
| 6 | 8.950 | 0.006 | New Alignment | Minor Bridge |
| 7 | 9.334 | 0.060 | New Alignment | Minor Bridge |
| 8 | 13.270 | 0.006 | New Alignment | Minor Bridge |
| 9 | 14.500 | 0.006 | New Alignment | Minor Bridge |
| 10 | 14.840 | 0.006 | New Alignment | Minor Bridge |

## Widening / Improvement of Culverts

There are many culverts on the project roads and these serve the purpose of cross-drainage works.  These are made of course rubble dry masonry abutments.  The top of the abutments is corbelled with a few layers of stones till the gap between the abutments is sufficiently reduced, when a stone slab is laid on the top.  Hand-packed stones are placed on the top and for the return walls of the scuppers. Retaining walls in course rubble dry masonry have commonly been provided on both ends of the scuppers.  It has been observed that the scuppers on the project road are performing satisfactorily and many of damaged/poor condition scuppers are under construction.

## Formation Width for New Bridges and Culverts

The formation width of structures shall be proposed to be maintained as full formation width of road section.

## Drainage Design

A good drainage system is vital for the safety and longer life of any structure. This is more relevant in the case of highways. Proper drainage of road surface, pavement and the foundation layers is basic requirement for maintaining the structural soundness and functional efficiency of a road. Pavement structure including sub grade must be protected from any ingress of water. For this purpose, the following conditions have to be ensured:

- Interception of the surface runoff;

- Keeping the water flow duration on the pavement to a minimum;

- Saving the pavement structure from stagnation of water;

- Efficient dispersal and disposal of water; and

- Quick disposal of sub-surface water away from the pavement.

Design for drainage is proposed to be carried out in accordance with the provision contained in IRC: SP 42-2014 and IRC: SP 50 -2013.

## Hydrological Design Methodology

For the calculation of discharge of the stream by Area-Velocity method, topographical survey including leveling surveys have been carried out across and along the water courses to determine the cross-section and the slope. A number of cross-sections have been taken at regular intervals on both upstream and downstream side of the structure, including one at the proposed location of the structure in accordance with IRC specifications.

The following assumptions have been made for peak discharge calculation:

For locations where water spreads over the banks, the cross-sections were extended up to the HFL, in order to calculate the effective cross-section of flow.

The longitudinal section to determine the bed slope have been taken following the channel course extending on both the upstream and the downstream sides of the structure.  Caution is taken by following the curved flow line for longitudinal gradient, rather than a straight line.

## Assessment of Peak Discharge

The peak discharge is calculated by the following method for cross section on the upstream and the downstream sections.

Area – Velocity Method (Kutter’s constant)

Q = A x V

V = C x (R x S)

Where,    Q = the discharge in cumecs;

A = Area of the cross section in sq. m.

V = Velocity in m/sec;

R = Hydraulic mean depth in m. = A / P;

P = Wetted perimeter of the stream in m.

C = Kutter’s constant which is given by

S = Bed slope of the stream; and

N = Co-efficient of rougosity which depends upon the roughness of the stream

The Design Discharge had been taken as the maximum of discharges at different cross sections. This will have 10% variations with one another.

## Hydraulic Analysis for Design HFL

HFL is fixed at the bridge location by local enquiry, then line parallel the bed slope line is drawn at this HFL, from this line HFL at different cross sections are found.

## Afflux Calculation

When the waterway area of the opening of a bridge is less than the unobstructed natural waterway area of the stream, i.e. when bridge contracts the stream, afflux occurs. The afflux will be calculated using Orifice formula as given below: -

Q=C₀ x √2g L Dd x √ {h+ (1+e) (U2/2g)}

Where, 	h = Afflux in meters;

Q = Discharge

U = velocity

L = Linear waterway

Dd = Depth at D/S side

W = width of River

Co and ‘e’ = Orifice formula co-efficient is taken from graph

## Scour Depth Calculation

To provide an adequate margin of safety for design of foundation, a further increase by 30% has been made over the design discharge as per IRC: 78-2000, to calculate mean scour depth.

#### By IRC: 5-1998 / IRC: 78-2000

#### As per IRC: 5-1998 or IRC: 78-2000, the mean depth of scour below the highest flood level, DSM, will be given by the following equation:

#### sm = 1.34 x (Db2 / Ksf) 1/3

Where, Db = the discharge in cumecs per meter width and Ksf = Silt Factor.

The value of ‘Db’ shall be the total design discharge divided by the theoretical effective linear waterway between abutments.

For most of the bridges, the silt factor, Ksf, has been calculated as per guidelines given in IRC-78: 2014since most of the bridges are Ghat section the bed material composes of pebbles and course sand for which silt factor assumed as 4.

## Maximum Depth of Scour for Design of Foundation

The maximum depth of scour below the Highest Flood Level (HFL) for the design of piers (dsmp) and abutments (dsma), having individual foundations without any floor protection are as follows:

Near pier:          		                          dsmp = 2 x Dsm

In the vicinity of abutment: 		dsma  = 1.27 x Dsm

## Vertical Clearance

Provision of vertical clearance in bridges above HFL shall be kept as per IRC SP-13, clause 12.3 as under.

| Discharge in m3/s | Minimum Clearance in m |
| --- | --- |
| up to 0.30 | 0.15 |
| Above 0.3 and up to 3.0 | 0.45 |
| Above 3.0 and up to 30 | 0.6 |
| Above 30 and up to 300 | 0.9 |
| Above 300 and up to 3000 | 1.2 |
| Above 3000 | 1.5 |

## Design Storm Calculation

The design of drainage system involves – (a) calculating the total discharge that the system will require to drain off and (b) fixing the slope and dimensions of the drain to have adequate capacity to carry the discharge and afford maintenance.

- Hydrological Design

Hydrological study is an important step prior to the design of road drainage system. Such analysis is necessary to determine the magnitude of flow and the duration for which it would last. Hydrological data required for design includes drainage area map, water shed delineation, arrow indicating direction of flow, outfalls, ditches, other surface drainage facilities, ground surface conditions, rainfall and flood frequencies.

To estimate the amount of runoff requiring disposal at given instant, information regarding rainfall intensities within the catchment area and the frequency with which this precipitation to assess peak run-off is essential. The ‘Rational Method’ is universally accepted empirical formula relating rainfall to run-off and is applicable to small catchment areas not exceeding 50 sqkm. The discharge is calculated by,

Q =	0.028 P A Ic

Where;

Q = 	Discharge (Peak run-off) in cum/ sec

P = 	Coefficient of run-off for the catchment characteristics

A = 	Area of catchment in Hectares

Ic = 	Critical intensity of rainfall in cm per hour for the selected frequency and for duration equal to the time of concentration

Coefficient of run-off ‘P’ for a given area is not constant but depends on a large number of factors such as porosity of soil, type of ground cover, catchment area, slope and initial state of wetness and duration of storm. For specific site conditions, the following values of ‘P’ given in IRC: SP 42-1994, ‘Guidelines on Road Drainage’ have been adopted.

Table 8.13 : Values of Coefficient of Run-off

| Sr. No. | Description of Surface | Coefficient of Run-off (P) |
| --- | --- | --- |
| 1. | Steep bare rock and water tight pavement surface | 0.90 |
| 2. | Steep rock with some vegetative cover | 0.80 |
| 3. | Plateau areas with light vegetative cover | 0.70 |
| 4. | Bare stiff clayey soils (impervious soils) | 0.60 |
| 5. | Stiff clayey soils with vegetative cover with uneven paved road surface | 0.50 |
| 6. | Loam lightly cultivated or covered and macadam or gravel road | 0.40 |
| 7. | Loam largely cultivated or turfed | 0.30 |
| 8. | Sandy soil, light growth, parks, gardens, lawns and meadows | 0.20 |
| 9. | Sandy soil covered with heavy bush or wooded/ forested areas | 0.10 |

The primary component in designing storm water drains is the design storm i.e. rainfall value of specified duration and return period. For the project road a return period of 25 years is considered to be adequate. As the extent of drainage system for the project road is small, even an intense rainfall of short duration may cause heavy outflows. The storm duration chosen for design purposes is equal to time of concentration. It has two components- (a) entry time and (b) time of flow. Because of lack of data for small duration peak rainfall for small catchments in project influence area, the following equation has been used to estimate the rainfall intensity for the shorter durations:

i=F(T+1)T(t+1)

where,

i= Intensity of rainfall within a shorter period of ‘t’ hrs within a storm

F= Total rainfall in a storm in cm falling in duration of storm of ‘T’ hrs

t= Smaller time interval in hrs within the storm duration in ‘T’ hrs

For the purpose of design storm, one-hour maps available from Directorate of Hydrology (small catchments), Central Water and Commission, New Delhi have been used. 1-hr rainfall for return period of 25 years for the project influence area has been taken as 100 mm.

- Design of Drain Section

For uniform flow in open channels, the basic relationships are expressed by the Manning’s Formula:

Q=1nAR2/3 S1/2

Where,

Q= discharge in cum/sec

n= Manning’s roughness coefficient

R= hydraulic radius in m which is flow cross section divided by wetted perimeter

S= energy slope of the channel which is roughly taken as slope of drain bed

A= Area of flow cross section in sqm

In design, the flow is assumed to be sub-critical. The slope and velocity are kept below the critical level. If design depth is less than critical depth, the section is to be redesigned to avoid critical flow situation.

To simplify the analysis the following energy slopes have been considered for the site-specific conditions:

- For longitudinal median drain : 1 in 200

- For lateral median drain and intersection drainage system : 1 in 285

- For side drains in urban areas : 1 in 200

- For side drains in plain terrain : 1 in 100

## Hydraulic Design and Resizing of Existing Culverts

Culverts like Slab culverts and Pipe culverts are predominant along the existing alignment. But they are neither sufficient in number not in terms of vent height at few locations. Hence as per Hydraulic designs per SP13:2004, re-sizing of culverts are proposed and improvement of culverts are presented vide Annexure 8.3.

## Slope Stabilisation and Protection Works

Erosion prevention is one of the major factors in design, construction and maintenance of highways. The most direct application of erosion control occurs in drainage design and in the writing of specifications for landscaping and slope planting. Erosion is minimized largely by the use of flat side slopes, rounded and blended with natural terrain; serrated cut slopes; drainage channels designed with due regard to width, depth, slopes, alignment, and protective treatment; inlets located and spaced with erosion control in mind; prevention of erosion at culvert outlets; proper facilities for groundwater interception; dikes, berms, and other protective devices to trap sediment at strategic locations; and protective ground covers and planting.

## Treatment of High Embankment

High embankment will be site specifically designed considering the quality of the available material, prevalent moisture condition and associated pore water pressure, bearing capacity of the founding strata and the requirement of any preloading etc. Stone pitching/gabion walls are proposed at these locations.

## Reinforced Earth Wall

The RE Wall has been provided as per required.

## Design Methods for Widening of Culverts

Longitudinal drains are designed in such a way that drains merges either at invert level of culverts or at bridge. Also, all culverts are proposed to be widened or reconstructed with full formation width of road. Project alignment is 2lane with paved shoulder hence full formation with will be 14.0m. In case of 4 lane section culverts are widened to full formation width of road including median, if any.

## Design Methods for Widening of Bridges

Longitudinal drains are designed in such a way that drains merges either at invert level of culverts or at bridge. Also, all bridges are proposed to be widened or reconstructed 11.0m width without foot path and 16m with footpath which is more than full formation width of road i.e. 14.0m for 2 lanes. In case of 4 lane section two separate bridges / ROB/ flyover are proposed with full formation width of 2 lanes for both side structures excluding median, if any.

## Designs for Road Side Drainage

Presence of a good drainage system is essential. It is therefore necessary to perform a detailed survey of the existing drainage system, the adjoining terrain and its slope, and recommendations for new drainage system or modification to existing drainage system.

Some basic principles have been adopted in order to meet IRC standards.

The surface water from the carriageway, the paved shoulders, the embankment slopes and the adjoining land must be effectively drained off without allowing it to percolate into the sub-grade.

The drains must have sufficient capacity and adequate longitudinal slope to drain away the entire collected surface water to the nearest natural surface stream, river or nallah.

No roadside drains are proposed where the longitudinal water bodies are present parallel to the road.  In the project alignment, the following types of drains will have to be proposed:

- Unlined Open Drain in rural section

- Lined Drain in urban areas

- Chute Drains

The hydraulic adequacy of the drains shall be checked as per IRC SP-42 “Guidelines on Road Drainage”. The design return period for the drains shall be taken as 25 years for median drains, chute drains, urban drains and other important drainage systems while the 2 years shall be taken as rural drainage system.

The rain water from the right of way of the road is ultimately required to be transported away before it can cause nuisance or damage. First of all, water has to be transported over the surface. This aspect has been well looked after by providing adequate cross-slope and compatible longitudinal profile. After running over the surface, most of the runoff is collected in the covered / open drain along the road. Open drains are preferred over covered ones as these are easier to maintain and allow removal of silt and other solids easily. Also, for a given cross section open drains can carry much larger discharge particularly in flood conditions where drain is surcharged.

## Unlined Open Drain in Rural Section

In rural areas where embankment height is less than 1.5m, open unlined toe drains and 1V: 2H side slope have been proposed near ROW on both sides of the road as per guidelines given IRC SP-42.

## Lined Drain in Urban Areas

In urban areas, water will flow across separators through cross cuts of size 150 cm x 150 cm top covered by precast slab in RCC M 20 grade provided at an interval of 10 m.  This will also facilitate crossings near building lines/built up areas. However, an attempt has been made to minimize such locations as low-level maintenance of covered drain is envisaged in post-construction phase. The design runoff has been considered not only from the road but also from the adjoining building lines.

## Drainage at Intersections

Any stagnation of water at intersections would reduce the capacity of junction resulting in queuing up of traffic. The level of junction has been kept higher than the cross roads so that water can reach the main drainage system which is along the main carriageway. No covered drain will be provided as these are likely to be choked due to sweepings from the road during the dry season. The side drain will have to be extended along the cross roads till the appropriate out-fall. In extreme cases, pipe drain will have to be proposed across the cross road to maintain the continuity of the drainage network if out-fall is not possible near-by due to site conditions.

## Drainage at Bridge

In case of bridges across a river, the main water is to be discharged into river bed through drainage spouts as per IRC standards. Properly designed filter media is to be provided behind abutment / earth retaining structures along with weep hole arrangement at 1.0 m interval to drain out the percolated water.

On approach portion longitudinal drains will have to be provided at the edges of roadway as kerb channel cum ditch drain. Kerb channel will be 55 cm wide having 6% slope and ditch will be of size 50 cm x 45 cm. Kerb channel will have RCC grating at 4.5 m interval to guide water into ditch. In initial stretch smaller depth, say 30 cm, can be adopted which then can be increased progressively to achieve 45 cm depth at the end of ramp.

## Road Markings, Signs and Other Safety Devices

## Road Markings

Road markings will be made for centre and edge lines using reflective thermoplastic

paints. Appropriate road markings will also be provided at junctions and crossings.

## Road Signs

Road signs are to place according to IRC: 67-2012. The signs are to be placed on embankment so that extreme edge of sign would be 2.0m away from the edge of the carriageway. The location of each sign is to be decided in accordance with the guidelines there in.

## Safety Barrier

Traffic barriers are protective devices that are placed between traffic and a potential

hazard off the roadway, with the intention of reducing the severity of a collision when an

errant vehicle leaves the travelled portion of the roadway. Barriers are to be provided at

high embankments, sharp curves and bridge approaches. The barrier is to be located at

the edge of paved shoulders.

## Miscellaneous Requirements

## Proposal for Truck Lay byes/Parking cum Rest Area

As per the detailed field surveys and reconnaissance, truck lay bye/ Parking cum rest areas are not proposed. The rest area will provide common facilities like petrol pump, first aid medical facilities, police office, restaurant and vehicle parking etc.

## Table 8.14: Truck lay byes/ Parking cum Rest Area Location

| Sr. No. | Existing Chainage |
| --- | --- |
| Nil |  |

## Pavement Design

The project road envisages two-laning with earthen shoulder and upgrading of the existing pavement to carry the anticipated traffic over the design period. This would involve:

- Construction of new pavement for widened and realigned/new alignment.

Flexible pavement is adopted for proposed new carriageway and reconstruction. Design period of 15 years considered for new carriageway.

## Traffic or Cumulative Equivalent Single Axle Loads

The project road is used by all types of vehicle with different loading and different axle configuration. For pavement design it is very necessary that all kinds of loads converted to a single common axle load hence using equivalent factor. The equivalent axle load factor (EALF) is based on a procedure of converting the number of repetitions of a given load into an equivalent number of repetitions of 8.16 tonne single axle load. The EALF based on fatigue cracking is different from that based on permanent deformation. The use of a single value for both modes of failure is approximate, at best. The most widely used method for determining the EALF is that which uses the empirical equations developed from the AASHTO Road Test, according to which the damage caused increases as the fourth power of the load. For example, a 10.2 tonne axle load would result in EALF of (10.2/8.16)4 = 2.5. Thus, an increase of 25 % in the axle load would result in 2.5 times more damage. This fact becomes even more significant in India where overloading is a norm. The fourth power relationship is internationally accepted and is used for design.

Equivalent single axle loads (ESALs) depends upon:

- initial traffic

- traffic growth (r)

- directional split of the traffic or directional distribution factor (DDF)

- number of lanes or lane distribution factor (LDF)

- axle load spectrum or vehicle damage factor (VDF)

Traffic surveys and subsequent analyses were carried out to determine the above parameters.

From the axle-load survey, VDF for each type of vehicle can be determined. The cumulative ESAL is calculated using the following equations:

i=j

∑

i=1

ESAL= 		Initial Traffic x 365 x (1+r)n−1r x Lane Factor x DDF x VDF

The equivalent single axle loads (ESALs) have been calculated assuming that the project road will be opened to traffic in the beginning of year 2021. Detail calculation of cumulative standard axle load in terms of million standards axle has been given in Annexure 8.4 and Design ESAL in Millions i.e. MSA for project road is presented as below:

Table 8.15: Summary of MSA

| Homogeneous Section |  | Existing Chainage |  | Design MSA (2021-2035) |
| --- | --- | --- | --- | --- |
| From | To | From | To |  |
| Chenani | Sudhmahadev | 0/000 | 22/713 | 20 |

For pavement design of project road, the above MSA values have been adopted. Pavement thickness is a function of log MSA, therefore, at high MSA values the change in pavement thickness is rather minor compared to the change in the MSA value.

## Shoulder

As per AASHTO, “as shoulder is the portion of the roadway contiguous with the travelled way for accommodation of stopped vehicles, for emergency use and for lateral support of sub-base, base and surface course." There should be continuous paved shoulder on both the right and the left side of all freeways facilities and the usable paved width of the shoulder should be between 10ft (3.048m) to 12ft (3.658m).

The factors affecting shoulder design are similar to those of mainline pavement design. The major difference is the amount of traffic. Traffic volume on shoulders is lower than on a mainline and much difficult to predict.

Three types of traffic may be considered in shoulder design:

- Encroaching traffic

- Parking traffic, and

- Regular traffic

Regular traffic is considered only if the use of shoulder as an additional lane for peak hour or detoured traffic is anticipated. If there is no regular traffic, the sum of encroaching and parking traffic is used to design the inner edge of shoulder adjacent to the mainline; while parking traffic is used to design the outer edge of shoulder. When there is a paved shoulder and no lateral obstruction within the shoulder area, trucks using the outer traffic lane tend to encroach on the shoulder. The percentage of parking traffic should be added to the encroaching traffic because any truck must encroach to park on the shoulder. It is a common practice to design mainline and shoulder pavements a single unit.

## Drainage

Design methods that develop pavement cross-sections on the assumption that the controlling factors are stress, strain, deformation and fatigue under repeated wheel loads, and ignore the effects of wheel load on water trapped in the pavement structure are a recipe for “designed to fail” pavement design. The trapped water in the pavement structure under the wheel loads generates pore pressures which drastically reduce the bearing capacity or strength of the granular layer and erodes the base and sub-base material, resulting in damage which may cause premature failure of the pavement.

To ensure adequate internal drainage of the pavement a full width of bottom most granular layer is proposed in the case of new rigid pavement, and a drainage layer under the rigid pavement has been provided.

## Flexible Pavement Structural Design for New Construction

## Recommended Pavement Design

Granular sub base should be laid in up to formation width. Similarly, a dense bituminous macadam thickness is proposed as per IRC design, would be most appropriate, and does not affect either design drastically. The recommended pavement design on project road, therefore, should consist of layer composition as per Table 8.16

Table 8.16: Recommended New Pavement Design

| Crust Composition for New Pavement as per IRC 37 - 2012 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Homogeneous Section | Chainage |  | CBR | MSA | Crust |  |  |  | S.Grade | Total Thickness |
|  | From | To |  |  | BC | DBM | WMM | GSB |  |  |
| 1 | 0/000 | 22/713 | 10 | 20 | 40 | 80 | 250 | 200 | 500 | 1070 |

## Reconstruction of Existing Pavement

- Reconstruction Sections are identified based on pavement condition survey, field observation and local inquiries for over topping during the field survey. Details for pavement condition survey are given in para of 4.2.2 of Engineering Surveys and Investigation chapter. Reconstruction sections of existing pavement are given with rehabilitate measures in Table 8.18.

- Table 8.18 Improvement Proposal for Reconstruction

| Sr. No. | Design Chainage |  | Length (Km) |
| --- | --- | --- | --- |
|  | From (Km) | To (Km) |  |
| 1 | 0.000 | 16.990 | 16.990 |
|  | Total Length |  | 16.990 |

## 8.11.7 Strengthening/Widening of Existing Pavement

Strengthening is not being done in any section of the road stretch.

- Table 8.21: Improvement Proposal

| Sr. No. | Design Chainage |  | Length (Km) | TCS Type | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | From (Km) | To (Km) |  |  |  |
| 1 | 0.000 | 0.800 | 0.800 | 3 | One side Valley & One side Built up or open area |
| 2 | 0.800 | 0.840 | 0.040 | 6 | Realignment |
| 3 | 0.840 | 0.890 | 0.050 | 4 | Approach to Bridge |
| 4 | 0.890 | 0.965 | 0.075 | Bridge | Bridge |
| 5 | 0.965 | 1.015 | 0.050 | 4 | Approach to Bridge |
| 6 | 1.015 | 1.274 | 0.259 | 2 | One side Hill & One side Valley (Drain on Hill Side) |
| 7 | 1.274 | 1.466 | 0.192 | 2 | One side Hill & One side Valley (Drain on Hill Side) |
| 8 | 1.466 | 2.400 | 0.934 | 6 | Realignment |
| 9 | 2.400 | 2.450 | 0.050 | 4 | Approach to Bridge |
| 10 | 2.450 | 2.700 | 0.250 | Bridge | Bridge |
| 11 | 2.700 | 2.750 | 0.050 | 4 | Approach to Bridge |
| 12 | 2.750 | 3.050 | 0.300 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 13 | 3.050 | 3.300 | 0.250 | 6 | Realignment |
| 14 | 3.300 | 4.100 | 0.800 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 15 | 4.100 | 4.167 | 0.067 | 6 | Realignment |
| 16 | 4.167 | 4.217 | 0.050 | 4 | Approach to Bridge |
| 17 | 4.217 | 4.223 | 0.006 | Bridge | Bridge |
| 18 | 4.223 | 4.273 | 0.050 | 4 | Approach to Bridge |
| 19 | 4.273 | 4.900 | 0.627 | 6 | Realignment |
| 20 | 4.900 | 5.400 | 0.500 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 21 | 5.400 | 5.558 | 0.158 | 6 | Realignment |
| 22 | 5.558 | 5.700 | 0.142 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 23 | 5.700 | 6.000 | 0.300 | 6 | Realignment |
| 24 | 6.000 | 6.185 | 0.185 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 25 | 6.185 | 6.400 | 0.215 | 6 | Realignment |
| 26 | 6.400 | 6.600 | 0.200 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 27 | 6.600 | 6.825 | 0.225 | 6 | Realignment |
| 28 | 6.825 | 7.400 | 0.575 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 29 | 7.400 | 7.447 | 0.047 | 6 | Realignment |
| 30 | 7.447 | 7.497 | 0.050 | 4 | Approach to Bridge |
| 31 | 7.497 | 7.503 | 0.006 | Bridge | Bridge |
| 32 | 7.503 | 7.553 | 0.050 | 4 | Approach to Bridge |
| 33 | 7.553 | 7.700 | 0.147 | 6 | Realignment |
| 34 | 7.700 | 8.677 | 0.977 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 35 | 8.677 | 8.727 | 0.050 | 4 | Approach to Bridge |
| 36 | 8.727 | 8.733 | 0.006 | Bridge | Bridge |
| 37 | 8.733 | 8.783 | 0.050 | 4 | Approach to Bridge |
| 38 | 8.783 | 8.800 | 0.017 | 6 | Realignment |
| 39 | 8.800 | 8.897 | 0.097 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 40 | 8.897 | 8.947 | 0.050 | 4 | Approach to Bridge |
| 41 | 8.947 | 8.953 | 0.006 | Bridge | Bridge |
| 42 | 8.953 | 9.003 | 0.050 | 4 | Approach to Bridge |
| 43 | 9.003 | 9.254 | 0.251 | 6 | Realignment |
| 44 | 9.254 | 9.304 | 0.050 | 4 | Approach to Bridge |
| 45 | 9.304 | 9.364 | 0.060 | Bridge | Bridge |
| 46 | 9.364 | 9.414 | 0.050 | 4 | Approach to Bridge |
| 47 | 9.414 | 10.200 | 0.786 | 5 | One side Hill & One side Valley (Breast Wall and Gabion Box Protection on hill side) |
| 48 | 10.200 | 10.600 | 0.400 | 6 | Realignment |
| 49 | 10.600 | 10.900 | 0.300 | 5 | One side Hill & One side Valley (Breast Wall and Gabion Box Protection on hill side) |
| 50 | 10.900 | 11.100 | 0.200 | 6 | Realignment |
| 51 | 11.100 | 11.400 | 0.300 | 5 | One side Hill & One side Valley (Breast Wall and Gabion Box Protection on hill side) |
| 52 | 11.400 | 11.625 | 0.225 | 6 | Realignment |
| 53 | 11.625 | 11.700 | 0.075 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 54 | 11.700 | 12.100 | 0.400 | 6 | Realignment |
| 55 | 12.100 | 13.217 | 1.117 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 56 | 13.217 | 13.267 | 0.050 | 4 | Approach to Bridge |
| 57 | 13.267 | 13.273 | 0.006 | Bridge | Bridge |
| 58 | 13.273 | 13.323 | 0.050 | 4 | Approach to Bridge |
| 59 | 13.323 | 13.555 | 0.255 | 6 | Realignment |
| 60 | 13.555 | 14.180 | 0.625 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 61 | 14.180 | 14.447 | 0.267 | 6 | Realignment |
| 62 | 14.447 | 14.497 | 0.050 | 4 | Approach to Bridge |
| 63 | 14.497 | 14.503 | 0.006 | Bridge | Bridge |
| 64 | 14.503 | 14.553 | 0.050 | 4 | Approach to Bridge |
| 65 | 14.553 | 14.700 | 0.147 | 6 | Realignment |
| 66 | 14.700 | 14.787 | 0.087 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 67 | 14.787 | 14.837 | 0.050 | 4 | Approach to Bridge |
| 68 | 14.837 | 14.843 | 0.006 | Bridge | Bridge |
| 69 | 14.843 | 14.893 | 0.050 | 4 | Approach to Bridge |
| 70 | 14.893 | 15.055 | 0.162 | 6 | Realignment |
| 71 | 15.055 | 16.300 | 1.245 | 1 | One side Hill & One side Valley (Breast Wall on hill side) |
| 72 | 16.300 | 16.700 | 0.400 | 6 | Realignment |
| 73 | 16.700 | 16.990 | 0.290 | 5 | One side Hill & One side Valley (Breast Wall and Gabion Box Protection on hill side) |
|  | Total Length |  | 16.990 |  |  |

- Details of TCS

| Summary of TCS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sr. No. | Detail | TCS | Length |  |
|  |  |  | (m) | (Km) |
| 1 | One side Hill & One side Valley (Breast Wall on hill side) | 1 | 6.902 | 6902 |
| 2 | One side Hill & One side Valley (Drain on Hill Side) | 2 | 0.451 | 451 |
| 3 | One side Valley & One side Built up or open area | 3 | 0.800 | 800 |
| 4 | Approach to Bridge | 4 | 1.000 | 1000 |
| 5 | One side Hill & One side Valley (Breast Wall and Gabion Box Protection on hill side) | 5 | 1.676 | 1676 |
| 6 | Realignment | 6 | 5.734 | 5734 |
| 7 | Bridge |  | 0.427 | 427 |
|  | Total Length |  | 16990.00 | 16.990 |