- Traffic Demand Forecast

  - Approach

For evaluating the benefits as well as costs incurred by the project roads, it is obvious that a certain period must be considered for the overall project. Though project once implemented has a long life, if a proper maintenance is carried out from time to time, it is also understood that the project will continue to benefit the society even after the expiry of the project period. For the present project, as mentioned in the TOR, period of 15 years has been considered for traffic demand forecasting.

Traffic demand forecast was carried out up to horizon year 2035. To calculate the growth rate for traffic projections, comparative analysis has carried out for all the methods. The methods used for growth rate calculation are as follows:

- Past trends in traffic growth (Vehicle registration Method)

- Econometric Model Method: IRC-108:1996

  - Past Trends in Traffic growth

There is no permanent count station along the project road.

  - Past trend in growth of Registered vehicle

The vehicle registration growth also gives an indication of the traffic growth. Vehicle Registration data of Jammu and Kashmir State has been taken for period 2004 - 2012. A growth rate for the same has been derived and the same has been shown in the Table 5.1 below.

Table 5.1: Growth Rate based on Vehicle Registration Method (Based on   Road Transport Year Book)

| Year | Goods | Growth Rate | Bus | Growth Rate | Car | Growth Rate | Three Wheelers | Growth Rate | Two Wheelers | Growth Rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2004- 2005 | 13949 |  | 20735 |  | 96590 |  | 15054 |  | 273265 |  |
| 2005 – 2006 | 16843 | 20.75 | 21435 | 3.38 | 109367 | 13.23 | 15919 | 5.75 | 297656 | 8.93 |
| 2006 – 2007 | 20004 | 18.77 | 22161 | 3.39 | 123357 | 12.79 | 16562 | 4.04 | 320754 | 7.76 |
| 2007 – 2008 | 22674 | 13.35 | 23149 | 4.46 | 139693 | 13.24 | 18440 | 11.34 | 341834 | 6.57 |
| 2008 – 2009 | 24768 | 9.24 | 24051 | 3.90 | 146562 | 4.92 | 19673 | 6.69 | 363029 | 6.20 |
| 2009 – 2010 | 43238 | 74.57 | 23480 | -2.37 | 183672 | 25.32 | 10958 | -44.30 | 407928 | 12.37 |
| 2010 – 2011 | 46792 | 8.22 | 25858 | 10.13 | 316539 | 72.34 | 12420 | 13.34 | 446791 | 9.53 |
| 2011 – 2012 | 51412 | 9.87 | 25765 | -0.36 | 255248 | -19.36 | 13759 | 10.78 | 480815 | 7.62 |
| Average Growth Rate |  | 22.11 |  | 3.22 |  | 17.50 |  | 1.09 |  | 8.42 |

  - Econometric Model Method (IRC-108:2015)

The traffic forecast by vehicle type has been carried out by adopting the transport demand elasticity method, which is a well-established and proven technique and is referred in India.

Elasticity of traffic demand is defined as the rate at which traffic intensity varies due to change in the corresponding indicator selected. Hence, to estimate the elasticity of traffic demand, it is necessary to establish the relationship between the growth in number of a given category of vehicle with one of the economic variables considered, such as NSDP, per capita income and population growth. Then the data can yield econometric model and the form of equation for estimation of traffic demand 	elasticity as recommended in IRC: 108-1996 of the following type:

Log (P) = Ao + A1Log (EI)

Where,

P=Number of vehicles

EI= Economic indicator

Ao=Constant

A1= a coefficient (elasticity value)

  - Past Trends in Economy and Population

The economic indicators of Jammu and Kashmir state, which are used for the regression analysis, are summed up in Table 5.2 below:

Table 5.2: Net State Domestic Products and Growth for Jammu and Kashmir  State (2004-2012)

|  | Jamm |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| YEAR | SECTORS |  | TOTAL NSDP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Population in lakhs | Per capita Income |  |  |  |
| 2004- 2005 | 107 | 21,734 | 2,329,200 |  |  |
| 2005 - 2006 | 109 | 22,406 | 2,437,109 | 107,909 | 4.63 |
| 2006 - 2007 | 110 | 23,375 | 2,579,432 | 142,323 | 5.84 |
| 2007 - 2008 | 112 | 24,470 | 2,738,731 | 159,299 | 6.18 |
| 2008 - 2009 | 113 | 25,641 | 2,910,203 | 171,472 | 6.26 |
| 2009 - 2010 | 115 | 26,519 | 3,051,315 | 141,112 | 4.85 |
| 2010 - 2011 | 117 | 27,666 | 3,225,589 | 174,274 | 5.71 |
| 2011 - 2012 | 118 | 29,067 | 3,431,596 | 206,007 | 6.39 |

As mentioned above, to establish elasticity of traffic growth, we have regressed past vehicle registration data with past economic indicators of the state. The ‘e’ values for the selected economic variables with respect to different vehicle types are shown in the Table 5.3 and are found with good fit, as reflected in their R2 values.

Table 5.3: Transport Demand Elasticity’s

| Vehicle Type | Independent variable | Elasticity Coefficient (e) | R2 |
| --- | --- | --- | --- |
| 2-Wheelers | Per Capita Income | 1.103 | 0.99 |
| 3-Wheelers | Per Capita Income | 1.124 | 0.99 |
| Car/ Jeep /Van/Taxi | Per Capita Income | 1.129 | 0.99 |
| Bus | Population | 1.113 | 0.99 |
| Trucks /Trailor | NSDP | 1.12 | 0.9 |
| Tractors | NSDP | 1.12 | 0.9 |

The relationship between different vehicle types and selected Per capita income (PCI) are presented in Figure 5.1.

Figure 5.1: Relationship between Vehicle Types and Per capita Income (PCI)

The comparison of vehicle growth rates by vehicle registration and econometric model is as shown in Table 5.4. It is appropriate to use the growth pattern that has emerged out of the economic model, which related the economic growth with the growth in vehicle registration data.

Table 5.4: Comparative Analysis and adopted of Growth Rates

| S.NO | Vehicle Type | Goods (%) | Bus (%) | Car (%) | 3-Wheeler (%) | 2-Wheeler (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Vehicle Growth Criteria Method | 22.11 | 3.22 | 17.50 | 1.09 | 8.42 |
| 2 | Net State Domestic Method | 6.17 | 4.72 | 4.79 | 4.63 | 4.68 |
| 3 | As per IRC | 5 | 5 | 5 | 5 | 5 |
| 4 | Adopted Growth Rates | 5 | 5 | 5 | 5 | 5 |

  - Estimation of Corridor Traffic and Projection

Consultant has adopted  growth rate of 5 % provided that annual rate of growth of commercial vehicles shall not be less than 5% for traffic projection and pavement design.

Batote to Kishtwar after the construction of Sudhmahadev tunnel and road stretch from Chenani to Sudhmaahdev, there could be increase in the traffic enroute to Doda Town and beyond. However, the proposed road configuration shall cater to the design service volume   9000(PCU). Further the road has been designed for 20 MSA against the projected 1.21 MSA (after 15 years)

Traffic demand projections for the horizon year (2035) on homogeneous sections are shown in Annexure 5. The following Table 5.5 shows the summary of projected traffic volume for homogeneous sections as per adopted realistic growth rates.

Table 5.5: Summary of Projected Total AADT Traffic PCU Volume/ day

| Homogeneous Section |  | Year 2017 |  | Year 2022 |  | Year 2025 |  | Year 2030 |  | Year 2035 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chenani to Sudhmahadev (Ch.0+000 to km 22+713) |  | 1256 |  | 1527 |  | 1856 |  | 2368 |  | 3023 |  |

  - Capacity Analysis and Level of Services

Capacity analysis is fundamental to the planning, design and operation of roads. It is a valuable tool for evaluation of the investment needed for the future improvements. The capacity figures used for determining the desired carriageway width in differing terrain w.r.t. traffic volume and composition are as per IRC: 64-1990. As per IRC 64:1990, it is recommended that on major arterial routes LOSB should be adopted for the design purpose. On other roads under exceptional circumstances, LOSC could also be adopted for design. For LOS C, Design service volume can be taken as 40 % higher than those for LOS B.

For augmentation of the facilities and up gradation of the project highway, the design service volume for the mountainous/hilly terrain condition and level of Service B & C is shown in Table 5.6.

Table 5.6: Design Service Volume for Different Lane Configurations

| Lane Configuration | Design Service Volume (PCUs per day) Level of Service B | Design Service Volume (PCUs per day) Level of Service C |
| --- | --- | --- |
| 2-Lane with 1.5m Paved Shoulder | 9000 | 10000 |
| 4-Lane with 1.5m Paved Shoulder | 10000 | 20000 |

  - Lane Requirements

Based on the assessment of the traffic demand on the various homogeneous sections of the Project Highway, the Consultant have carried out detailed option analysis for Two- laning with paved shoulders. Based on the estimated Capacity & Design Service Volume, the number of lanes required for the project road is worked out for LOS B which is presented in Table 5.7 below.

Table 5.7: Lanning Requirement for the Project Corridor

| Homogeneous Sections | LOS B | LOS C |
| --- | --- | --- |
|  | 2-Lane with Paved Shoulder | 2-Lane with Paved Shoulder |
| Chenani to Sudhmahadev (Ch.0+000 to km 22+713) | 2018 | Beyond Analysis Period |

  - Lane Improvement Proposals

Capacity analysis and lanning requirements have been carried out separately for the homogeneous sections as per the traffic demand and travel characteristics.

It is revealed from the capacity analysis results and considering future growth, the project road requires 2 lanes with Paved shoulder for capacity augmentation and efficient movement of traffic up to project horizon year 2035 for the above mentioned Homogeneous sections .

  - Intersection Improvement Proposals

Intersections are an important part of the highway because it controls the efficiency, the safety and the capacity. All intersections falling on the project corridor have been studied for the improvement to allow a safe connection to the corridor and minimum interference to the through traffic. The traffic on the connected road for major intersections have been studied and projections have been made for its future development. Before recommending the improvement, all available options in order of their importance as enumerated ahead have been considered:

- At grade Intersections

- Grade separated Intersections

- Major junction with acceleration and deceleration lanes;

- Major junction with channelization of traffic or Rotary;

- Minor Junction as ‘Left In & Left Out’

However, no the intersection volume count survey has been carried out during identified peak periods.

at Chalan

at Battal

  - Pedestrians Crossing Facilities

Pedestrian movement along any road is always expected near built up areas, bus bays and intersections. Safe crossing facilities for pedestrians are proposed at major intersections and bus bays. These facilities are planned in accordance with the relevant provisions contained in IRC-11, IRC-17 and IRC-103. At intersections, controlled form of crossing is achieved through provision of 3 m wide zebra crossing, accompanied by STOP line. Pedestrian guard rail has also been proposed at locations to safe guard the pedestrian movement at urban locations.