CHAPTER-1

- introduction

  - General

The project highway section of NH-37, starts at km 535+250 (Design km 534+800) and ends at km 581.700 (Design km 580.778) of Demow – Bogibil Junction Section in the state of Assam. The project section traverses mainly through Sibsagar and Dibrugarh districts.

The existing project road is having a 2-lane carriageway with average width of 7.0m, width of earthen varies on either side are 1.0m to 2.0m. The existing pavement is flexible type. The road is passing through the built-up areas of towns and villages enroute. Important towns along the alignment are Demow, Moran and Dibrugarh town. The condition of the pavement varies from Good to Poor. The height of embankment is in range of 1.0m to 1.5m except on the approaches to bridges where height of embankment varies from 7m to 8m. The existing Right of Way varies from 18 m to 60 m. The average ROW is 30m.

  - Salient Features of Project Road

The existing project road is two lane flexible pavements throughout its length. There are three major bridge, 1 minor bridge, and 38 nos. of culverts along the Project Highway. Out of 38 culverts, 2 nos. of Hume Pipe and 36 nos. of RCC box culverts. 3 nos. of vehicular underpasses and 2 nos. of cattle underpasses have been proposed on the Project Highway section of NH-37 between km 535.250 to km 581.700. Service road has been proposed to be provided on all the habitat portions falling on this stretch. Number of the villages where service road has been provided total length of the service roads, on both sides of the highway, works out to 9.500 km length of 7 m width of carriageway.

  - The Work of Consultancy

The work for consultancy services for Feasibility and Detailed Project Report for four/six laning of Jorhat –Dibrugarh section (length 147 km) of NH-37 in the State of Assam under NHDP Phase III Programme awarded to Scott Wilson Kirkpatrick India Pvt. Ltd. Vide NHAI (Ministry of Road Transport and Highways) letter no. NHAI/BOT/11012/55/2005/322 dt. 05/08/2005. The date of commencement is 5th August 2005.

1.4	Scope of Consultancy Service

Scope of consultancy services is to prepare detailed project report for the Demow – Bogibil junction (km 535.250 to km 581.700) project reach based on necessary investigation and submit necessary data, reports, designs, drawings and documents for selecting suitable concessionaires for their execution through Private Participation under BOT system.

1.5	Objective of Detailed Projects Report (DPR)

The objective of DPR is basically to provide upgrading requirement for existing feature and requirements for additional carriageways, Detailed Designs of Pavements & structures including their GADs, Detailed designs of intersections and detailed economic analysis giving Prioritization of Project road sections. The services also included updating of traffic survey, preparation updated cost estimate, LA report for BOT option and preparing BOT bid document on new MCA for civil contractors.