# CHAPTER– 6

## Cost Estimate

## 6.0	Cost Estimate

## General

6.1.1	Construction costs are an important component of a Detailed Project Report as it determines the funding requirement and also provides vital inputs of economic evaluation. The construction cost is influenced by the suggested specifications of construction materials, physical characteristics of the terrain traversed, geometric standards, land-use, bridges etc.

6.1.2  	Projects cost estimates have been prepared based on various items of works required for the rehabilitation and up-gradation work. The item rates for roadworks have been taken from Assam PWD, schedule of rates (SOR) 2009-2010 and updated with 5% escalation for 2010-11. The items not included in the SOR have been taken from prevailing market rates.

## Material Investigations

The sources for borrow material, metal quarry material and sand quarry have been identified and samples were tested to determine their suitability of various components of the work.

## Borrow and Quarry Material

The borrow areas were identified in and around (not within ROW) and along the project road. The details of borrow area locations, distance from the project road, quantity available are given Table 6.1.

Table 6.1:  Details of Borrow Area along the Project Corridor

| Sl No. | Chainage/Location | Distance from Existing Road (Km) | Sample Taken |
| --- | --- | --- | --- |
| 1 | 509.000 LHS | 3.00 | Yes |
| 2 | 524.000 LHS | 2.00 | Yes |
| 3 | 554.000 LHS | 4.00 | Yes |
| 4 | 556.000 LHS | 4.00 | Yes |
| 5 | 571.000 RHS | 2.00 | Yes |
| 6 | 580.000 LHS | 3.00 | Yes |
| 7 | 587.000 LHS | 1.00 | Yes |
| 8 | 604.000 LHS | 6.00 | Yes |

## Metal Quarry

The sources of road aggregates (metal) have been identified within the project influencing area.  The materials were tested to ascertain quality for its suitability in construction. The locations, name of quarry /village and approximate distances are presented in Table 6.2

| Table 6.2: Details of Quarry Material (Metal) along the Project Corridor |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sl No. | Chainage/Location | Distance from Existing Road (Km) | Sample Taken | Type of Material Available |
| GSB (Material) |  |  |  |  |
| 1 | 403.000 LHS | 15.00 | Yes | 53mm, 40mm, 20mm, 12mm, 6mm and dust |
| 2 | 403.000 LHS | 58.00 | Yes | 53mm, 40mm, 20mm, 12mm, 6mm and dust |
| 3 | 516.000 LHS |  | Yes | 53mm, 40mm, 20mm, 12mm, 6mm and dust |
| 4 | 574.000 LHS |  | Yes | 53mm, 40mm, 20mm, 12mm, 6mm and dust |
| Aggregate (Stone) Material |  |  |  |  |
| 1 | 359.000 LHS | 6.00 | Yes | 53mm, 40mm, 20mm, 12mm, 6mm and dust |
| 2 | 359.000 LHS | 6.00 | Yes | 53mm, 40mm, 20mm, 12mm, 6mm and dust |
| 3 | 403.000 LHS | 78.00 | Yes | 53mm, 40mm, 20mm, 12mm, 6mm and dust |
| 4 | 403.000 LHS | 105.00 | Yes | 53mm, 40mm, 20mm, 12mm, 6mm and dust |
| 5 | 516.000 LHS | 35.00 | Yes | 53mm, 40mm, 20mm, 12mm, 6mm and dust |
| 6 | 555.000 LHS | 70.00 | Yes | 53mm, 40mm, 20mm, 12mm, 6mm and dust |

## Estimation of Quantities

6.3.1	The quantities of all major items of work proposed for the Project Road – namely, the carriageway, shoulder and median have been estimated on the basis of the typical cross section, detailed plans and designs furnished in this Report.

6.3.2	The total earthwork in cut-and-fill has been determined from computerised CAD software.

6.3.3	The rates for different categories of land have been ascertained from the Revenue Department.

## 6.4	Cost Estimates

The project road start at Demow (km 534.800) and end at Bogibil Junction (km 580.778) to be developed as four lane configurations. The project cost includes, widening and strengthening of existing carriageway, reconstruction of existing carriageway and the Moran bypass has been proposed as new 4 lanes with flexible pavement. The Total Project Length is 45.978 km.

The Project cost has been prepared item-wise as per details given below:

| Sl. No. | Description | Amount in Rs. | Amount in Crores (Rs.) |
| --- | --- | --- | --- |
| A | Civil Works | 45.98 | 45.98 |
| 1 | Road Works | 2407938910 | 240.79 |
| 2 | Culverts (Box & Pipe culverts) | 107411191 | 10.74 |
| 3 | Structures i.e. Bridges, Flyover, Underpass including RE Wall | 469000698 | 46.90 |
| 4 | Service Road | 132497432 | 13.25 |
| 5 | Drainage & Protection Works | 116476728 | 11.65 |
| 6 | Toll Plaza and junction | 85000000 | 8.50 |
| 7 | Miscellaneous items | 91082199 | 9.11 |
| 8 | Truck lay-byes & Bus-bays | 25102764 | 2.51 |
|  | Total Civil Works as per SOR 2009-10 | 3434509922 | 343.45 |
|  | Update Cost (5% escalation for year 2010-11) | 3606235418 | 360.62 |