CHAPTER – 7

EXECUTIVE SUMMARY – EIA & EMP

7.0	EIA & EMP

  - Introduction

The present project of four laning of the existing section of National Highway-37 from Demow–Bogibil Junction (km 535.250 to km 581.700) is intended to improve the transport movement in the regions as well as economic and social welfare of people. The purpose of road widening is to increase the road capacity, road safety, improve pavements, reducing the travel time, lowering the cost of vehicular maintenance and creating opportunities for the growth of markets, jobs, education, health, etc.

The report is project specific and covers the environmental and socio-economic issues within ROW, though their impacts have been assessed up to the aerial distance of 10 km on both the sides of the highway stretch.

  - Description of the Project

The project road is a part of NH-37 from km 535.250 (Demow) to Km 581.700 (Bogibil Junction). NH-37 which runs almost parallel to Brahmputra river divides the Assam state into two regions Northern and Southern. NH-37 spread across the whole state from West to East.  NH-37 starts at Panchratna in Goalpara districts and traversing about 700kms, it terminates at Saikhoa Ghat. At Tinsukhia it branches out to NH-38.

The project road from Km 535.250 (Demow) to Km 581.700 (Bogibil Junction) runs in flat terrain. The total alignment is sprayed with habitation all along. The major towns are Demow, Moran and Dibrugarh (Dist HQ).

The acquisition of land for widening the existing alignment and improving its geometry will be a major social impact as this will lead to loss of land (agricultural, residential and commercial), structures (residential, commercial and community), income and livelihood and community and cultural sites.

The acquisition of land will be essential for 4 laining of the project road. Compensation for land and structures will be given as per National Highways Act (Amendment) 1997 and National Policy of Resettlement and Rehabilitation (NPRR)-2004, Government of India.

  - Objectives

The objectives of the study:

- Identification of Valued Ecosystem Components (VECs) – ecological, social, economic and cultural – and the impact of widening of road on them

- Analysis of baseline data on various components of the environment

- Determination of the magnitude of environmental impacts so that due consideration may be given to them during planning, construction and operational phases of the project implementation.

- Assessment of the socio-economic conditions of the Project Affected Persons and suggestions for their improvement.

- Identification of areas and aspects which are environmentally or socio-economically significant / insignificant.

- Submission of environmental enhancement plan and environmental management plans for enhancing & mitigating the negative impacts.

- Development of the road alignment in such a way that the environment and settlements are least affected.

  - Description of the Environment

The physical, biological and socio-economic baseline environmental conditions along the project road are discussed in the following sub sections:

# A.	Physical Resources

Topography and Physiography: The terrain all along the project road is mostly flat located in Brahmaputra valley. The physiography of the area is constituted by a variety of elements such as flood plain and swamps.

The project road is sprayed with large habitation all along the project road. The total alignment is sprayed with habitation all along. In about 30 km of length tea gardens exist both sides along the project road. Towns’ enroute make the road very congested and at many places ROW is encroached by villagers.

Surface and Ground Water Hydrology: Being close to the river Brahmputra, there are many tributaries and channels leading the high water table all along the project road.

Most of the settlement areas, along the project road have ponds and rain water is stored in these ponds for bathing and animal usages in non-monsoon months.

Ground water is the dominant water source in the area. The source of recharging of ground water is mostly from precipitation (rainfall) and from flowing water bodies (tributaries of Brahamputra). In the area groundwater is drawn mostly by hand pumps for domestic purpose and by tube wells for agricultural purpose. Static water levels vary along the stretch of project road. First or upper ground water aquifer lies in the range of 15 to 20 m below ground level (bgl).

Soil Characteristics: The soils of the area are basically the products of the fluvial processes of the Brahmaputra and its tributaries. The plains are composed of alluvium, which may be classified as new and old. The new alluvium varies mostly from clayey to sandy loam in texture and is slightly acidic in reaction. It is deficient in phosphoric acid, nitrogen and humus, but rich in lime and potash. It is found in the vast plain of the district along the river valleys, especially in their lower courses. The old alluvium on the other hand occurs in the upper and middle parts of the valleys.

The soil found in the area along the project road is recent alluvial soil and mostly sandy loam in texture. The colour of soil along the project road, vary from brown to yellowish to brown. Soil of the area are mostly suitable for paddy crop, jute, mustered and potato. Soil of the area are acidic in nature.

Water Quality: Ground water samples were collected from most commonly used water sources along the project road. The colour of water samples is less than <1 Hazan unit and odour is unobjectionable. Turbidity in surface water samples was found to be 1.5 to 2.0 NTU (Nephelometric Turbidity Unit). In the water samples, pH values were found in the range of 7.1 to 7.5. Total hardness values vary from 133 to 156 mg/l. Iron contents in the water samples vary from 0.6 to 0.9 mg/l. Chloride contents range between 14.2 to 18.2 mg/l. It is observed from results that all essential parameters of water samples are well below the prescribed limit for potable water as per IS: 10500.

Total dissolved solids (TDS) in the water samples range between 322 to 402 mg/l. Calcium contents lie in the range of 13.6 to 21.2 mg/l. Sulphate contents vary in between 5.3 to 7.2 mg/l. Nitrate contents vary in the range of 3.2 to 5.2 mg/l. NH4N and TKN were found not detectable.  On perusal of the analysis data, it is observed that analyzed values of water samples are well below the prescribed limits for drinking water as per IS: 10500 for all the parameters analyzed.

Climatology:  The area experiences subtropical monsoon climate with mild winter, warm and humid summer. Rainfall decreases from south to north and east to west in the area. The temperature On the basis of the climatic characteristics such as distribution of temperature, rainfall, rainy days, humidity, presence of fogs and thunderstorms, the climate of the area may be classified into four seasons:

(a) winter, (b) pre-monsoon, (c) monsoon and (d) retreating monsoon

The wind speed in the area varies from 1.7 to 5.4 kmph during the year. High wind speed is observed April to July (5.0 to 5.4 kmph) while November and December observe relatively low wind speed (1.7 to 1.9 kmph).

Ambient Air Quality: Ambient air quality monitoring has been carried at Demow, Moran and Dibrugarh locations along the project road. The baseline data of air environment was monitored for, Suspended Particulate Matter (SPM), Respirable Particulate Matter (RPM), Sulphur dioxide (SO2), Oxides of Nitrogen (NOX) and Carbon monoxides (CO). National air quality standards for rural and residential area, along the project road are most of the time met for monitored parameters (RPM, SO2, NOx and CO) for mostly all AAQM locations.

Noise Measurements: Noise measurements have been carried at Demow, Moran and Dibrugarh locations along the project road. Measured Leq noise levels are most of the time, within the limit stipulated for residential area and commercial area at all the locations.

B. 	Sensitive Locations

Archaeological and Historical Monuments: Three historical monuments are located along the project road.

Historical Monument Located along the Project Road

| Sl. No. | Name | Chainage | Details | Location |
| --- | --- | --- | --- | --- |
| 1 | Antique Graves of Aohm Kings | Km 577.950 | Graves of Aohm Kings: Dihinga Borua Modam and Alam Dihingia Baiz Barua | LHS |

These monuments have been completely avoided in widening plan and will be not be affected.

Religious Structures: Many religious structures are located along the project road within proposed ROW of 60 m, close to the project road. The alignment has been finalized in such away that religious structures will not be affected by 4 lanning of the project road.

Wildlife Sanctuaries: There is no wildlife sanctuary in the area.

## C.	Screening of Potential Environmental Impacts and Mitigation Measures

i. 	Impact on Physiography and Topography

Since proposed project is only 4 laning of existing road, impact on the physiography and topography of the area would be insignificant during construction and operation phase. The design will consider the improvement of roadside drainage conditions through the improvement of cross-drainage structures. Design of the cross drainage structures will follow IRC Guidelines (IRC, 1995).

ii.  	Potential Environmental Impacts on Soil

a. 	Design and Construction Phase

Loss of Productive Soils:  Some loss of productive soils is anticipated because at places the land along the project road is agricultural land. However, the productivity of crops in the region will not be affected by the project road. Efforts will be made to minimize acquisition of productive land. The location of construction camp and other construction sites shall not be located on productive agricultural land. The topsoil from areas to be permanently covered shall be stripped to a specified depth of 150 mm and stored in stockpiles. The stored topsoil will be utilized for the redevelopment of borrow areas, top dressing of the road embankments, fill slopes and filling up of tree pits proposed as part of compensatory plantation. Temporary loss of productive soil is likely if haul roads for the transport of borrow materials, traffic detours during construction, etc. are routed through agricultural lands.

Soil Erosion : During the upgrading of the project, some trees, shrubs and grasses will need to be cleared along the project road, which may pose some soil erosion problem during first few rains. Suitable mitigation measures will need to be implemented to prevent the soil erosion problem.

Contamination of Soil: In the project road, the contamination of the soil will be negligible. Further, the contractor shall initiate measures to minimize waste generation from all construction activities. At construction sites, the vehicles and equipments will be maintained properly and refueled only at fueling areas, without any spillages.

Compaction of Soil: During construction phase, at some places, soil in the adjoining productive lands beyond the ROW may be compacted by the movement of construction vehicles, machinery and equipment.

##### D.	Mitigation Measures

To mitigate the adverse impact during excavation of borrow pits, following mitigation measures shall be taken:

- Indian Road Congress (IRC): 10 -1961 guideline should be followed for excavations of earth from borrow areas.

- Borrow areas shall be excavated as per the intended end use by the owner. In some cases the owners of land want to develop the area in to pond for rearing fishes. The following criteria have been used for selection of borrow pits and amount that can be borrowed. They are as follows:

- Borrow areas should not be located on cultivable lands. However, if it becomes necessary to borrow earth from temporarily acquired cultivated lands, their depth should not exceed 45 cm. The topsoil to a depth of 15 cm shall be stripped and set aside. Thereafter, soil may be dug out to a further depth not exceeding 30cm and used in forming the embankment.

- Borrow pit shall be selected from wasteland at least 500m away from the road;

- Priority should be given to the borrowing from humps above the general ground level within the road land;

- Priority should be given to the borrowing by excavating/enlarging existing tanks;

- Borrowing should be from land acquired temporarily and located at least 500m away from the road;

- Borrowing should be from mounds resulting from the digging of well and lowering of agricultural fields in vicinity of the road;

- In case of settlements, borrow pit shall not be selected within a distance 800m from towns or villages. If unavoidable, earth excavation shall not exceed 30 cm in depth.

- The haulage distance from site should not be too far.

- Aggregate required for road construction shall be procured from quarries approved by Assam Pollution Control Board. Air and noise emissions from quarry shall be well within the prescribed limit.

### E. 	Operation Phase

No impact is anticipated on soil during the operation phase and therefore, no mitigation measure is required. However, soil erosion shall visually be checked on slopes and high embankment areas along the road during operation phase. In case soils erosion is observed suitable measures should be taken to control the soil erosion.

#### iv.  	Impacts on Water Resources

#### a.  	Design and Construction Phase

4 laning of the project road is not likely to have any significant impact on existing drainage system along the project road. Further, adequate culverts and bridges for movement of runoff during rains will be provided at required locations.

Mitigation Measures

The contractor shall ensure that construction debris does not find it way in to the minor drainage channels which may get clogged. To restore the surface water flow/drainage, proper mitigation measures will be taken along the road, like:

- Drainage line will be constructed all along the project road.

- Extra culverts will be proposed to accommodate the drainage requirement along the alignment.

- Drainage arrangements will be suggested in respect of site conditions in the form of drainage layer and sub-surface drains in the full width of formation or below the shoulder so as to keep the pavement well drained at locations where these are required.

- Good engineering and construction practiced should be followed.

#### b. 	Operation Phase

During the operation phase, drainage pattern or hydrology of the area will not be affected. Therefore, no impact is anticipated during operation phase and no mitigation measures are required.

v.  	Impact on Water Environment

a. 	Design and Construction Phase

During design and construction phase, drainage pattern and run off flow conditions along the project road will not be significantly affected. Water requirement for the project road will be temporary and meet through existing surface/ground water sources available place to place along the road. Therefore, no impact is anticipated on the water resources of the area.

No local water supply will be used for upgrading/construction purposes. Hence, the impact on the local water supply will be insignificant.

#### vi.	Mitigation Measures

Following mitigation measures are suggested to mitigate any adverse impacts during design and construction phase:

- Provision shall be made for proper drainage along the road.

- Construction camps, if any, shall be properly located to avoid contamination of surface water bodies by the generation waste and waste-water.

- Good engineering practices to be followed to avoid the clogging of water channels along the project road.

- Water to be used for construction shall have separate source.

a. 	Operation Phase

During operation phase, water quality, drainage pattern and run off flow conditions along the project road will not be significantly affected and no impact is anticipated on water quality along the project road. Therefore, no mitigation measure is required for operation phase.

#### vii. 	Impact on Ambient Air Quality

a. 	Construction

Certain amount of dust and gaseous emissions will be generated during the construction phase from excavation machines and road construction machines. Pollutants of primary concern include Suspended Particulate Matter (SPM) and Respirable Suspended Particulate Matter (RSPM). However, suspended dust particles may be coarse and will be settled within a short distance of construction area. Therefore, impact will be temporary and restricted within the closed vicinity of the construction activities along the road only.

The following mitigation measures will also be taken to mitigate the dust entrainment and fugitive emissions from the various sources:

- Asphalt and hot-mix plants will be located at least 1 km away in down wind direction from inhabited urban and rural stretches along the road with the clearance from Assam Pollution Control Board.

- Sprinkling water will control fugitive dust emissions.

- Sprinkling of water on the dust prone areas and construction yard.

- Regular maintenance of machinery and equipment will be carried out.

b. 	Operation Phase

During the operation phase, air quality along the road will be affected by vehicular emissions on the project road. Pollutants of primary concern will include NOx, CO, HC, SPM, RPM and SO2. It is suggested that roughness of project road should be as per IRC:SP:16-2004. As per IRC guidelines roughness <2000 mm/km is considered good for bituminous concrete surface.

vii.	 Impact on Noise

a.	Construction Phase

During the construction phase, the noise level is bound to increase by the use of construction machines, generators, etc. However, these noise levels will be temporary in nature mostly during daytime only. The impact of noise on surrounding area during the construction phase will be limited to short distance, hence inferred as moderate.

Some salient features related to potential noise impact of road development include:

- The road noise impact is greatest where road passes though populated areas,

- The range of noise level should be understood in relation to the habitation type also, for example, road noise in industrial areas is not likely to be problematic but at sensitive location like schools, worship place, its impact may be significant

b.	Operation Phase

During the operation phase, noise will be generated through the vehicles movement. It will depend up on traffic density, number of traffic event. Plantation along the road and improved road conditions will be helpful in reduction on noise levels during operation phase. To mitigate the impact of noise levels during operation phase, following mitigation measure are anticipated:

- It is suggested that roughness of project road should be as per IRC:SP:16-2004. As per IRC guidelines roughness <2000 mm/km is considered good for bituminous concrete surface.

- Developing trees barriers between the road and sensitive area, wherever it is possible.

- Noise measurements should be carried out along the road to ensure the effectiveness of mitigation measures.

#### viii. 	Impact on Flora, Fauna and Ecosystem

a. 	Design and Construction Phase

During the design and construction of project road, vegetation in the form of trees, shrubs and grasses will be cleared. Matured trees are growing along the road within the ROW, which will need to clear for construction of road.

Important positive and negative impacts on flora, fauna and ecosystem due to project road are:

- As an estimate approximately 21117 trees will need to be cleared for construction of project road and will be compensated through compensatory afforestation as per The Forest Conservation Act, 1980.

- There will be no loss of bio-diversity since no rare/endangered plant or animal species is going to be eliminated due to the proposed 4 laning of project road.

- There will be no loss of animal habitat by the project road itself, borrow pit areas and quarry sites.

- The proposed project road will not affect migratory path of animal breeding zone or important ecosystems.

- The aquatic ecology will not be damaged due to construction and operation phase of project road, as most of the water bodies remains dry during non-monsoon months, Further, suitable bridges and structures will be provided on these water bodies.

- The loss of trees and herbal cover at least during the construction phase, is likely to produce some negative impacts.

ix.	Impact of Dust Generation of Vegetation

During construction of road, dust will be emitted and settled on the foiler surface of leaves of plants in the close vicinity of project road. It may interfere plant growth for short duration as this dust will be washed off during first rain.

Mitigation Measures

4 laning for project road shall be prepared in such a way that trees cutting can be minimised up to maximum possible extent. In eccentric widening, one side plantation shall be utilised in median it will greatly reduce the number of tree to be cut.

Careful and proper planning can be done for re-plantation of trees during design and right at the commencement of construction and the phase wise removal of growing trees will mitigate the negative impacts.

Compensatory plantation can be started during construction phase parallel to the construction activities.

Operation Phase

During the operation phase, no adverse impact is anticipated on the flora and fauna of the area. Plantation along the road on available space will enhance flora in the area.

Mitigation Measures

Compensatory afforestation should be done along the project road during operation phase, where ever it is possible. Monitoring of survival of trees should be done at regular interval and suitable mitigation measures should be taken to protect the trees.

x.  	Impacts on Social Environment

Construction and operation phases of project road will have some beneficial impact on social environment. Some increase in income of local people is expected as local unskilled, semiskilled and skilled persons may gain direct or indirect employment during construction phase. Since the immigration of work force during construction phase is likely to be very small, the social impacts on literacy, health care, transport facilities and cultural aspects are expected to be insignificant.

## a.	Environmental Monitoring Plan

An institutional mechanism needs to be incorporated in the proposed project management and execution system. The NHAI will be responsible for the implementation of all the mitigation and management measures suggested in EMP for project road. The NHAI is also responsible for implementation the complete resettlement and rehabilitation for all those affected by the project and is committed to ensure the income restoration of the project affected and project displaced families.

The NHAI has certain organizational and institutional capacity to be able to satisfactory complete the implementation of the EMP and RAP.

#### b.	Environmental Monitoring Plan

To ensure the effective implementation of the mitigation measures and environmental management plan during construction and operation phase of the project road, Environmental monitoring plan has been designed for Ambient Air Quality (AAQ) Monitoring, Water Quality Monitoring, Noise Levels Monitoring, Soil Erosion and plantation areas.

  - Environmental Budget for Mitigation Measures & Environmental Management Plan

Summary of approximate cost for implementation of mitigation measures and environmental management plan (EMP), environmental monitoring & training is given below:

| Sl. No. | ## Various Activities | Cost (Rs.) |
| --- | --- | --- |
| 1 | ## Implementation of EMP | 26,716,154.00 |
| 2 | ## Compensatory Afforestation and Maintenance | 21,933,566.00 |
| 3 | Environmental Monitoring Cost | 1,155,524.00 |
| 4 | Environmental Training and Mobilization | 898,741.00 |
|  | ## Total | 50,703,985.00 |

  - Resettlement Action Plan

The proposed widening of NH-37 between Demow–Bogibil junction (km 535.250 to km 581.700) will require displacement of residential, commercial and some other structures besides clearing cultivable land and trees. The existing ROW all along the road is about 30 m except in few stretches where it varies. For widening the existing 2 lane road to 4 lane dual carriage way configuration, it has been proposed to provide for a ROW of 60 m in general.