Chapter – 5

BRIDGES & CULVERTS

5.0     	Bridges & Culverts

  - Bridges

Parallel bridges are proposed to be built either on upstream side or on downstream side as per site requirement. Wherever feasible as per site requirement span arrangement have been kept similar to the existing bridges. In some cases, a combination of two/three spans only one span to cover the stream has been adopted.

Table 5.1 gives the existing and proposed bridges on the section along with proposed span configuration.

Table 5.1 : Existing and Proposed Major/Minor Bridges

| SL. No | Bridge No. | Location in km | Type of bridge | River / Nalla / Rd | Span arrangement No. x Span (Existing) | Skew / Curve / Straight | Length (m) | Proposed Span arrange-ment No.xSpan | Pro-posed Deck Width (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 561/1 | 562.200 (E) 561.375 (D) | Major Bridge | Teloi | 2x 20.12 +1x 28.66 | Straight | 68.90 | 19.33+26.508 +19.33 | 12.00 |
| 2 | 567/1 | 567.680 (E) 566.822 (D) | Minor Bridge | Dulla Demow | 2x10.80 +1x 9.80 | Straight | 31.40 | 1 x 29.248 | 12.00 |
| 3 | 572/1 | 573.310 (E) 572.420 (D) | Major Bridge | Buri Dehing | 4 x 37.61 +2 x 42.97 | Straight | 236.38 | 2x35.458 +2x40.97 +2x 35.458 | 12.00 |
| 4 | 576/1 | 577.094 (E) 576.180 (D) | Major Bridge | Sessa | 2x 40.48 | Straight | 80.96 | 2x38.48 | 12.00 |

  - Culverts

The table given below indicates the existing CD works with widening/new construction requirement. The recommendations need to be verified at site and detail designs developed for each case in consultation with independent consultant. The details of each type of culvert, number of culverts require repair and widen and reconstruction is given in Table 5.2.

# Table 5.2: Details of Culverts and Treatment

| Type of Culvert | Number of Culverts and Condition |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Existing 2 lane from km 535.250 to 581.700 |  |  | Construction of Culverts on new 2 lanes |  |
|  | Existing | Repairs / Retain and Widen | Reconstruction | Type of Culvert proposed | No |
| Hume Pipe | 2 | - | 2 | Hume pipe | 2* |
| Box | 36 | 35 | 1 | Box | 36* |
| Total | 38 | 35 | 3 |  | 38 |
| * 7 Numbers of culverts (2 Hume Pipe & 5 Box) on road passing through Moran town have been included |  |  |  |  |  |

| Sl. No. | Location | Design km | Proposed Structure | Proposed Size Arrangement | Proposed Total Width (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | Moren Bypass | 550.545 | Pipe | 1x1.2m | 4-lane |
| 2 |  | 550.839 | Pipe | 1x1.2m | 4-lane |
| 3 |  | 551.098 | Pipe | 1x1.2m | 4-lane |
| 4 |  | 551.266 | Pipe | 1x1.2m | 4-lane |
| 5 |  | 551.476 | Pipe | 1x1.2m | 4-lane |
| 6 |  | 551.871 | Box | 1 x 1.0 x 1.5 | 4-lane |
| 7 |  | 552.120 | Pipe | 1x1.2m | 4-lane |
| 8 |  | 552.338 | Pipe | 1x1.2m | 4-lane |
| 9 |  | 552.754 | Pipe | 1x1.2m | 4-lane |
| 10 |  | 558.331 | Pipe | 1x1.2m | 4-lane |
| 11 |  | 558.685 | Pipe | 1x1.2m | 4-lane |
| 12 |  | 559.569 | Pipe | 1x1.2m | 4-lane |
| 13 |  | 560.341 | Pipe | 1x1.2m | 4-lane |
| 14 |  | 560.396 | Pipe | 1x1.2m | 4-lane |
| 15 |  | 560.696 | Pipe | 1x1.2m | 4-lane |
| 16 |  | 561.590 | Pipe | 1x1.2m | 4-lane |
| 17 |  | 562.417 | Pipe | 1x1.2m | 4-lane |
| 18 |  | 562.789 | Pipe | 1x1.2m | 4-lane |

  - Vehicular and Cattle / Pedestrian Underpass

The Concessionaire shall provide 3 nos. of vehicular underpasses and 2 nos. of cattle underpass of adequate size of 12 m x 5.5m and 5.0m x 3.5m along the Project Highway. The details are given in the Table 5.3.

Table 5.3: Details of Proposed Underpasses

| Sl. No. | Design km | Description | Name / Location |
| --- | --- | --- | --- |
| Vehicular Underpasses |  |  |  |
| 1 | 550.200 | Vehicular Underpasses | Moran |
| 2 | 552.400 | Vehicular Underpasses | Jatiany |
| 3 | 564.100 | Vehicular Underpasses | Khowang Ghat |
| Cattle underpass |  |  |  |
| 1 | 549.880 | Cattle underpass |  |
| 2 | 561.550 | Cattle underpass |  |

The concessionaire shall take up ‘Detailed Engineering Study’ to ascertain further details of all intersections based on conceptual designs as suggested above. The treatment at the intersections shall be designed in accordance with the latest IRC / MoRT&H guidelines spelt out in Schedule-D.

  - Grade Separator (2 lane Flyover)

The Concessionaire shall provide 1 nos. of Grade Separator of adequate size of 1x30 along the Project Highway as per locations given in the Table 5.4.

Table 5.4: Details of Grade Separator

| Sl. No. | Design km | Description | Name / Location |
| --- | --- | --- | --- |
| 1 | 579.880 | Bogibil | NH-52A |