CHAPTER - 4

HIGHWAY DESIGN

4.0	Highway Design

  - Introduction

    - General

In case of flexible pavements, the detailed pavement design requirements of the Project Highway and that of the service roads shall be in accordance with Schedule D.  Pavement shall be flexible for main carriageway, paved shoulders, service roads, truck lay byes, bus bays and for cross roads up to ROW limits and invariably rigid at toll plaza locations.

    - Pavement Composition for New Carriageway

Flexible Pavement

The design traffic is then computed for design life location wise is presented in chapter 3.

Table 4.1 : Design Traffic in Million Standard Axles

| Design Period | Demow to Start of Dibrugarh Bypass |
| --- | --- |
| Years | Design Traffic (MSA) |
| 10 | 35 |
| 15 | 60 |
| 30 | 173 |

CBR Value

The average CBR value of existing subgrade soil for the entire stretch is considered as 7%.

The flexible pavements would be designed as a multi-layer system consisting of typical component layers, namely sub-base, base course, binder course and surface course. The flexible pavement thicknesses required for new 2 lanes and bypasses as given in Table 4.2.

## Table 4.2:  Proposed Flexible Pavement Thickness

## (For Widening, New 2 lanes, 4 lane Moren Bypass)

| Chainage (km) |  | Design Traffic (MSA) | CBR of Subgrade (%) | For 10year design life Pavement Thickness (mm) |  |
| --- | --- | --- | --- | --- | --- |
| From | To |  |  | Wearing Course, BC (mm) | Binder Course (mm) |
|  |  |  |  |  | DBM |
| 534.80 | 580.770 | 35 | 7 | 40 | 115 |

| Chainage (km) |  | Design Traffic (MSA) | CBR of Subgrade (%) | For 15year design life |  |
| --- | --- | --- | --- | --- | --- |
| From | To |  |  | Granular Base, WMM (mm) | Granular Sub-base, GSB (mm) |
| 534.80 | 580.770 | 60 | 7 | 250 | 230 |

Overlay Thickness

The overlay thickness calculated and proposed is presented in Table 4.3.

Table 4.3: Proposed Overlay Thicknesses

| Chainage (km) |  |  | Proposed Composition and Thicknesses |  |
| --- | --- | --- | --- | --- |
| From | To | Length, km | BC, mm | DBM, mm |
| 534.80 | 551.00 | 16.20 | 50 | 70 |
| 551.00 | 567.00 | 16.00 | 50 | 70 |
| 567.00 | 582.60 | 15.60 | 50 | 70 |

Rigid Pavement

The designed thickness of rigid pavement for Toll Plaza and Truck Laybye obtained from IRC: 58 is presented in Table 4.4.

## Table 4.4: Proposed Rigid Pavement Design for Toll Plaza and Truck lay-bye

| Material Type | Layer Thickness in mm |
| --- | --- |
| Pavement Quality Concrete | 300 mm |
| Dry Lean Concrete | 150 mm |
| Granular Sub-base | 150 mm |
| Sub-grade | 500 mm |

Pavement for Service Roads

Service roads with 7.0m width of carriageway on both sides have been proposed at urban or semi urban locations to facilitate the movement of the local traffic. The pavement thickness as computed with the above considerations is presented in Table 4.5.

## Table 4.5: Flexible Pavement Thickness required by IRC: 37-2001 –For Service Roads

| Design Traffic (MSA) | CBR of Sub-grade (%) | Pavement Thickness (mm) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Wearing Course, SDBC (mm) | Binder Course, DBM (mm) | Granular Base, WMM (mm) | Granular Sub-base, GSB (mm) | Total (mm) |
| 5 | 7 | 25 | 50 | 250 | 180 | 505 |

Paved Shoulder Composition

The paved shoulder shall be designed as an integral part of the pavement for the main carriageway. Therefore the total pavement thickness in the paved shoulder would be the same as in the carriageway.

  - Cross Sectional Details

    - Lane Width

The width of a basic traffic lane is proposed to be 3.50m. Thus, for 2-lane the carriageway widths will be 7.0m. In case of the divided cross-section, a 0.25 m wide edge strip with a composition same as that of the adjacent pavement will be provided as compensation for kerb shyness.

    - Paved Shoulders

Full strength pavement for paved shoulder is proposed. Width of these shoulders will be 1.50m in rural sections. However in urban section the width has been increased to 2.0m. These will provide better traffic operations, lower maintenance cost and will help to directly use these as part of carriageway when the road is widened in future

    - Earthen Shoulders

Earthen shoulders are proposed to be 2.0m wide on either side of the proposed main carriageway for rural pain and rolling terrains. If site condition warrants, the width can be reduced to 1.50m for urban /built-up sections.

    - Median

In rural stretches, the project road will have raised median and its width is proposed to be 4.50m. However in urban stretches 1.5m wide median with pedestrian guard rail has been proposed. The medians will be edged with 250 mm high non-mountable concrete kerbs.

    - Service Road

Where Service road is provided, it will be of 5.5m/7.0m carriageway width of different pavement composition based on the expected traffic.

    - Side Slopes

For fill sections, the following side slopes are proposed:

| Embankment height up to 3.0 m | - | 2H:1V and Considering the Embankment material property |
| --- | --- | --- |
| Embankment height from 3.0 m to 6.0 m | - | 2.5H:1V and Considering the Embankment material property |
| Embankment height exceeding 6.0 m | - | To be designed as per material property and as per IRC: 75 |
| Cutting sections | - | 1H: 1V and with considering the material property of the cutting material. |

    - Crossfall

The crossfall for the pavement and paved shoulders will be 2.5%.  For earthen shoulders and median the corresponding value will be 3.0%.

    - Tentative Widening Scheme for 4-Laning

| TCS | Widening Scheme | Length in km |
| --- | --- | --- |
| TCS-2: | Length of Left Widening | 10.628 |
| TCS-1: | Length of Right Widening | 16.050 |
| TCS-3: | Length of Four Lane Bypass/Realignment | 13.350 |
| TCS-4: | Length of Concentric Widening with Service Road | 1.100 |
| TCS-6: | Length of RHS Widening with Service Road | 1.650 |
| TCS-7: | Length of 2-Lane Approach with RE Walls | 0.800 |
| TCS-8: | Length of 4-Lane Approach with RE Walls | 2.400 |

| Sl. No. | Design Chainage From | Design Chainage To | Length (km) | C/s Type |
| --- | --- | --- | --- | --- |
| 1 | 534.8 | 535.2 | 0.4 | TCS - 6 |
| 2 | 535.2 | 535.8 | 0.6 | TCS - 1 |
| 3 | 535.8 | 536.1 | 0.3 | TCS – 3 |
| 4 | 536.1 | 537.9 | 1.8 | TCS – 1 |
| 5 | 537.9 | 538.25 | 0.35 | TCS – 3 |
| 6 | 538.25 | 540.8 | 2.55 | TCS – 2 |
| 7 | 540.8 | 541.1 | 0.3 | TCS – 3 |
| 8 | 541.1 | 541.9 | 0.8 | TCS – 2 |
| 9 | 541.9 | 542.7 | 0.8 | TCS – 6 |
| 10 | 542.7 | 546.1 | 3.4 | TCS – 1 |
| 11 | 546.1 | 546.55 | 0.45 | TCS – 3 |
| 12 | 546.55 | 548.7 | 2.15 | TCS – 1 |
| 13 | 548.7 | 549.15 | 0.45 | TCS – 6 |
| 14 | 549.15 | 549.8 | 0.65 | TCS – 2 |
| 15 | 549.8 | 550.6 | 0.8 | TCS – 8 |
| 16 | 550.6 | 552 | 1.4 | TCS – 3 |
| 17 | 552 | 552.8 | 0.8 | TCS – 8 |
| 18 | 552.8 | 557.3 | 4.5 | TCS – 3 |
| 19 | 557.3 | 557.8 | 0.5 | TCS – 4 |
| 20 | 557.8 | 560.9 | 3.1 | TCS – 3 |
| 21 | 560.9 | 561.5 | 0.6 | TCS – 4 |
| 22 | 561.5 | 561.7 | 0.2 | TCS – 3 |
| 22 | 561.7 | 562.3 | 0.6 | TCS – 3 |
| 23 | 562.3 | 562.7 | 0.4 | TCS – 2 |
| 24 | 562.7 | 563.7 | 1 | TCS - 1 |
| 25 | 563.7 | 564.5 | 0.8 | TCS – 8 |
| 26 | 564.5 | 567.8 | 3.3 | TCS – 1 |
| 27 | 567.8 | 570.3 | 2.5 | TCS – 2 |
| 28 | 570.3 | 570.7 | 0.4 | TCS – 3 |
| 29 | 570.7 | 572.6 | 1.9 | TCS – 2 |
| 30 | 572.6 | 572.9 | 0.3 | TCS – 3 |
| 31 | 572.9 | 573.4 | 0.5 | TCS – 2 |
| 32 | 573.4 | 574.3 | 0.9 | TCS – 1 |
| 33 | 574.3 | 574.6 | 0.3 | TCS – 3 |
| 34 | 574.6 | 574.9 | 0.3 | TCS – 1 |
| 35 | 574.9 | 575.05 | 0.15 | TCS – 3 |
| 36 | 575.05 | 575.65 | 0.6 | TCS – 1 |
| 37 | 575.65 | 576 | 0.35 | TCS – 3 |
| 38 | 576 | 576.35 | 0.35 | TCS – 2 |
| 39 | 576.35 | 577 | 0.65 | TCS – 3 |
| 40 | 577 | 579 | 2 | TCS – 1 |
| 41 | 579 | 579.4 | 0.4 | TCS – 2 |
| 42 | 579.4 | 580.2 | 0.8 | TCS – 7 |
| 43 | 580.2 | 580.778 | 0.578 | TCS - 2 |
| Total Length |  |  | 45.978 |  |

    - Intersections

There are three major junctions across the Project Highway. For improvement of major intersections Flyover / Underpasses are to be provided. Minor intersections shall be developed at grade as per IRC standards.   The approach roads are basically MDR & ODR along the project road. The details of junctions are given in Table 4.6.

Table 4.6: Details of Junction and Proposal

| Sl. No. | Chainage (KM) | Location | Type | Cross Road |  | Proposal |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Leading to | Lane Configuration |  |
| 1 | 535.570 | Demow Junction | 3-Legged | Demow Town | Intermediate lane | At Grade |
| 2 | 580.789 | Bogibil Junction | 3-legged (NH-52 A) | Bogibil Bridge | 2-lane | Flyover (2-lane) |

  - Drainage

Following Measures shall be adopted:

- Side ditches of required cross-section for area drainage on both sides of carriageway in rural sections.

- Covered pucca pre-cast RCC segmented drains underneath the sidewalk/footpath in urban sections.

- Pipe drains & outlets to drain water from elevated structures..

- Chute drains along with shoulder drains in high embankment (3m and above);

- U-shaped Median drains at super elevated sections with proper outfall connections;

  - Service Roads

Service roads with 7.0 m width of carriageway have been proposed at urban or semi urban locations excepting the bypass stretch to facilitate the movement of the local traffic. The location of the service roads to be provided is listed below in Table 4.7

Table 4.7:  Proposed Service Road Locations

| Sl. No. | Existing Chainage |  | Design Chainage |  | Length (km) | Width (m) | Side |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | From | To | From | To |  |  |  |
| 1 | 535.250 | 535.670 | 534.800 | 535.200 | 0.40 | 7 | Both Side |
| 2 | 542.300 | 543.100 | 541.900 | 542.700 | 0.80 | 7 | Both Side |
| 3 | 549.165 | 549.620 | 548.700 | 549.150 | 0.45 | 7 | Both Side |
| 4 | 550.240 | - | 549.800 | 550.600 | 0.80 | 7 | Both Side |
| 5 | Moran Bypass |  | 557.300 | 557.800 | 0.50 | 7 | Both Side |
| 6 | 562.310 | 562.530 | 561.500 | 561.700 | 0.20 | 7 | Both Side |
| 7 | 564.560 | 565.380 | 563.700 | 564.500 | 0.80 | 7 | Both Side |
| 8 | 580.300 | 581.100 | 579.400 | 580.200 | 0.80 | 7 | Both Side |

  - Truck Lay-bye

The provision of truck lay-byes shall be governed by site requirements and parking demand and as per the guidelines of MoRT&H Technical circular No. RW/34032/5/88 PO-II dated 22.8.88. Parking shall be designed in the form of rectangle/trapezoidal area parallel to and separated from the carriageway by a physical barrier. The parking lots shall have necessary facilities like repairing, eating, resting, fuelling and shall be suitably landscaped. Location of Lay Bye is given in Table 4.8.

###### Table 4.8 : Proposed Location of Truck Lay-Byes

| - Sl. No. | - Proposed Lay-byes locations |  |
| --- | --- | --- |
|  | - Chainage | - Side of Carriage way |
| - 1 | Km 562.650 | - Both Side |

0.17	Bus Bays

The concessionaire shall provide Bus Bays at 2x4 nos. of Bus bays along the Project Highway. The locations of these bus bays are given in Table 4.9 are tentative and shall be finalized by the Concessionaire in consultation with the IE/NHAI.

###### Table 4.9 : Proposed Location of Bus Bay

| Sl. No. | Chainage | Side | Name / Location |
| --- | --- | --- | --- |
| 1 | Km 542.000 | Both | Sakadia |
| 2 | Km 555.620 | Both | Moran |
| 3 | Km 563.200 | Both | Khowang |
| 4 | Km 580.250 | Both | Borbarauah |